🔹 Part D: CMOS Logic

- Why does CMOS inverter consume almost no static power?

- Explain why NMOS passes weak logic ‘1’ and PMOS passes weak logic ‘0’.

- What is the advantage of CMOS inverter over NMOS-only inverter?

- Why is PMOS usually designed wider than NMOS in CMOS circuits?

- What is the truth table of a 2-input NAND gate implemented in CMOS?

🔹 Part E: Noise, Delay, and Power

- Define Noise Margin High (NMH) and Noise Margin Low (NML).

- If VOH = 4.9V, VOL = 0.1V, VIH = 3.5V, VIL = 1.5V, calculate NMH and NML.

- Define propagation delay of a CMOS inverter.

- What is the difference between rise time (tr) and fall time (tf)?

- What causes short-circuit current during switching?

🔹 Part F: Advanced Insights

- Why is the bulk terminal tied to source in discrete MOSFETs?

- What happens if MOSFET bulk is left floating in an IC?

- Explain skew in CMOS inverters and its effect on frequency.

- Define fan-out. Why does it increase delay?

- State two ways to reduce propagation delay in a CMOS inverter.