| Topic | Details |
| --- | --- |
| is voltage regulation for DC only or AC or both ? | 🔹 DC Voltage Regulation When we say “voltage regulator” in electronics (78xx, LM317, LDO, buck, etc.), we are almost always talking about DC voltage regulation. Use case: - Powering microcontrollers, CPUs, sensors, FPGAs, LEDs, etc. Goal: - Keep DC output constant even if input changes, load changes, or temperature changes. Examples: - 7805 → 5 V DC regulated output - Buck converter → 12 V DC → 5 V DC 🔸 AC Voltage Regulation (less often discussed in electronics) AC voltage can also be regulated, but the circuits are very different. Purpose: - Keep AC RMS value constant even when input mains voltage fluctuates. Used in: - Voltage stabilizers for homes - Servo stabilizers - Automatic Voltage Regulators (AVR) for generators - Uninterruptible power supplies (UPS) AC voltage regulation is not the focus in analog/digital IC design because: - Almost all electronic circuits run on DC internally. - Even AC mains input first gets rectified + filtered → DC, and then regulated. Why your whole learning path is DC-focused Because in real electronics and VLSI: AC mains (230 V / 110 V) ⬇ Rectifier + Filter → Unregulated DC ⬇ DC Voltage Regulator → Clean stable DC for circuits Whether it's a microcontroller board or a SoC: 🔹 Inside chips → everything runs on DC 🔹 Power rails are DC (1.2 V, 1.8 V, 3.3 V, etc.) 🔹 Clock is AC-like waveform, but supplied by DC power Final takeaway Type; Regulates; Used for. DC voltage regulation; Stable DC output; Electronics, embedded systems, VLSI. AC voltage regulation; Stable AC RMS input; Mains stabilizers, generators, UPS. So — voltage regulation exists for both AC and DC, but when we talk about voltage regulators in electronics, it almost always means DC regulation. |
| What does a voltage regulator do? | Link : https://youtu.be/AX5SqSJu6R4?si=6lO8gsnXo1aeNSzp - What is an unregulated power supply - This section is called unregulated power supply - Here the output DC is almost stable - There are several factors that affects output DC volt (makes it change) - If the load changes – the DC volt also changes (example : if low resistance value load is connected – the volt output will be low as the volt drop there is very low) - If the input AC volt changes (or you can say line volt changes) - Temperature – it changes the current flowing through the diodes - A VOLTAGE REGULATOR corrects all 3 problems - Independent of the load current (IL) [ independent of the load ] - Independent of the AC line volt - Independent of temperature - There are two types of regulators - Linear regulator - Switching regulator |
| Voltage Dropout / Dropout voltage concept | Dropout Voltage (Vdrop) is the minimum difference between the input voltage and the regulated output voltage for which a linear regulator can still maintain proper regulation. In simple words: How close can Vin get to Vout while the regulator still works? 🧠 Why does this matter? A linear regulator needs some extra voltage headroom across itself to control the output. If the input voltage falls too close to the desired output voltage, the regulator loses control → Vout starts dropping instead of staying constant. 📌 Formal definition Vdropout=Vin(min)-Vout So, the regulator will work only when: Vin≥Vout+Vdropout 🌟 Example (Normal Linear Regulator) 7805 → regulated 5 V output Typical dropout voltage ≈ 2 V Vin(min)=5V+2V=7V This means: - If input = 9 V → works - If input = 7.5 V → works - If input = 6.8 V → FAILS (output will drop below 5 V) 🌟 Example (LDO — Low Dropout Regulator) LM1117-5.0 → 5 V regulator Dropout ≈ 1.1 V @ 800 mA load Vin(min)=5V+1.1V=6.1V LDOs are designed to have a very small dropout voltage. ❗ Why does dropout happen? Because in a linear regulator: - The pass transistor (PNP / NPN / PMOS) needs some voltage across it to stay in the active region. - If Vin becomes too close to Vout → transistor goes into saturation → regulation fails. 🚦 Real-world meaning Case; What happens. Vin > Vout + Vdrop; Regulator works perfectly. Vin ≈ Vout + Vdrop; Output unstable, may ripple. Vin < Vout + Vdrop; Vout decreases (no regulation). 💡 Dropout voltage depends on load Higher load current → regulator needs more headroom → dropout increases. Datasheets usually give dropout curves like: Dropout vs Load Current 🧠 Super short memory line Dropout is the minimum “extra voltage” a linear regulator needs across it to keep Vout stable. Final punchline Regulator type; Typical Dropout. Normal linear (e.g., 7805); ~2 V. LDO; 0.1–1.2 V. Switching regulator; No dropout concept (works even if Vin ≈ Vout or Vin < Vout depending on topology). |
| Linear Vs switching regulator | Link : https://www.youtube.com/watch?v=abaTptF4LD0&list=WL&index=6 |

| Linear Regulators | Link : https://www.youtube.com/watch?v=abaTptF4LD0&list=WL&index=6 https://www.youtube.com/watch?v=SdkS1k6GcG8&list=WL&index=3 A linear regulator controls voltage by acting like an adjustable resistor. It drops extra voltage as heat to keep the output constant. Example analogy : Note : - linear regulator output : always lesser than input volt (it cannot increase the volt) [like switching regulators do] - they can’t boost the volt because of their way of operating and internal structure 🔹 How it works (concept) If input is 12 V and you need 5 V: - The regulator does not convert extra voltage into energy. - It simply blocks / drops 7 V across itself. The current through regulator and load is the same, so power loss : Pheat=(Vin-Vout)×Iload So, it is simple, low-noise, stable, but not efficient when Vin is very higher than Vout or Iload is high. 🔹 Key points Feature; Linear Regulator. Efficiency; Low to moderate. Heat generation; High (wastes power as heat). Output noise/ripple; Very low (clean). Complexity; Very simple. Common examples; 7805, 7905, LM317, LDO. Where used - Analog circuits - Audio circuits - Microcontroller boards - FPGA core rails (with small dropout) |
| --- | --- |
| LDO (low dropout regulator | Link : https://www.youtube.com/watch?v=SdkS1k6GcG8&list=WL&index=3 An LDO stands for Low Dropout Regulator. It is a special type of linear voltage regulator that can maintain a stable output even when the input voltage is only slightly higher than the output voltage. 🔥 Why is it called “Low Dropout”? Because it has a very low dropout voltage. Dropout voltage (recap) The minimum difference Vin − Vout required for proper regulation. A normal linear regulator (like 7805) need: ≈ 2V dropout → Vin must be at least Vout + 2V But an LDO can work with dropout as low as: ≈ 0.1V to 1.0V → Vin can be just slightly above Vout Examples Regulator; Dropout; Result. 7805; ~2 V; Needs Vin ≥ 7 V for 5 V output. LM1117 (LDO); ~1.1 V; Can give 5 V from 6.1 V. Ultra-modern LDOs; ~0.3 V; Can give 3.3 V from 3.6 V. ⚙️ Why can LDOs work with such low dropout? Because of the type of pass transistor, they use internally: Regulator type; Pass element; Dropout ability. Normal linear regulator; NPN transistor; Higher dropout. LDO regulator; PNP or PMOS transistor; Very low dropout. PNP / PMOS does not require a large headroom across the device to stay in active region → low dropout. 🧠 Big picture summary Type; Efficiency; Dropout; Ripple; Complexity. Normal Linear Regulator (78xx); Low; High (~2 V); Very low; Very simple. LDO (Low Dropout Regulator); Low; Low (0.1–1.0 V); Very low; Simple-moderate. Switching Regulator; High; N/A; Higher; Complex. 🔍 When to use an LDO Use an LDO when: - Input voltage is barely higher than desired output voltage - Battery powered devices (battery voltage drops during use) - When low noise and low ripple are needed (unlike switching regulators) Examples - 3.7 V Li-ion battery → 3.3 V logic rail → LDO is ideal - 5 V USB → 3.3 V analog circuitry → LDO gives very clean output - FPGA / MCU analog reference rails → LDO preferred 🛑 When not to use an LDO If the voltage difference × load current creates too much heat. Example: Vin = 12V, Vout = 5V, Iload = 0.5A Power dissipated = (12 – 5) × 0.5 = 3.5 W Not good → overheating Better to use buck converter in such high-power drops. ⭐ One-line memory An LDO is a linear regulator designed to work with very small input–output voltage difference. It gives clean, low-noise power in battery and sensitive circuits. |
|  | - 2 types of linear voltage regulator - Shunt regulators / parallel regulators - Series regulators - SHUNT Regulators/parallel regulator (controls current flow) - Shunt regulator is connected in parallel with load - Shunt regulator reduces the volt appearing at the load (it doesn’t possess the ability to increase volt – so thus unregulated power supply output should be in such a way that it always is high and if required shunt regulator can lower it down - This regulator does so by providing lower resistance path to pass more current through it (lowering current at load) - series Regulators (controls voltage drop) - series regulator controls the voltage drop across itself - if it senses the volt at load is decreased – then it lowers its voltage drop (that in turn increases voltage available at load - if it senses the volt at load is increases – then it Highers its voltage drops (that in turn decreases voltage available at load] |
| SHUNT Regulators/parallel regulator (controls current flow) | Link : https://www.youtube.com/watch?v=5vec422YMik&list=PLXb3r5ny8_1VGu4f8FoKp4ZkdEyd8CF1a&index=2 Zener diode shunt regulator Normal diode Zener diode Zener diode equivalent circuit Zener shunt regulator : - as we discussed in the prev section (shunt regulator is parallel to load ) - shunt regulator controls the current through it – that is how they change the voltage of load\ - voltage across load is same as zener voltage(zener breakdown voltage) [ VZ=VL ] - this zener volt regulator is not very efficient – ex. if the voltage source is high and the regulated volt we need is low (RZ & RS ) dissipates power in terms of heat in order to bring the volt down [ for more details – you can visit diode section of the learning] Shortcomings of shunt regulators - series resistance might have to absorb a lot of current in order to achieve the op voltage - zener diode might need to carry lot of current in order to maintain the op voltage |
| Line and Load regulation | Link : https://www.youtube.com/watch?v=eADfk2EN1MA&list=PLXb3r5ny8_1VGu4f8FoKp4ZkdEyd8CF1a&index=3 - VL = Load volt - Vi = input volt (line volt / AC volt) - Here when current is 0 (meaning no load condition – the Vout = VNL=no load volt ) ( here u can also consider no load = infinite resistance (which makes op volt appear full)) - Even when the load changes and current changes – ideal regulator should have no change in volt appearing across load - Here when the load is max (current is seem to be decreasing ) and the output appears is called Vout = VFL=full load volt ) ( here u can also consider full load = lowest resistance or the min resistance (which makes op volt appear little less than VNL)) |

| Series voltage regulators | Link : https://www.youtube.com/watch?v=J8obBcHviaY&list=PLXb3r5ny8_1VGu4f8FoKp4ZkdEyd8CF1a&index=4 Shortcomings of shunt regulators which we need to overcome in series regulators - series resistance might have to absorb a lot of current in order to achieve the op voltage - zener diode might need to carry lot of current in order to maintain the op voltage Basic series voltage regulator - here – the zener diode breakdown volt or zener volt is what creates the reference voltage/output volt - same as zener shunt regulator where it was the output voltage - voltage at base is fixed by zener diode (which is Zener’s breakdown volt) + also for silicon diode in active region the base emitter junction drop is +0.7v (so the op volt Vout= Vz-0.7 - in series regulator the current passes through series resistance (Rs) – but here current passes through transistor (which is the key feature) - in series regulators very short value of current flows through series resistor (Rs) but in shunt regulator entire current needs to be passed through series resistor (Rs) [ so this regulator is more efficient and doesn’t burn up much power as a shunt regulator ] - also keep in mind that we have base current that goes to transistor - Here major factors we shall look upto - IE (emitter current) is basically our load current - And all transistor has some β (current gain) - Which means to connect a load consuming X value of current – we need to ensure our IB is such a way that it can fulfil IE - Shortcomings of simple series regulator - A simple series regulator uses zener diode to provide reference voltage for output and the output is considered as zener volt – BE junction volt (=0.7v) but that isn’t true (it is not const ) current shows different exponential trend (till 0.7v it is 0 and after 0.7v it increases exponentially (making Vbe change)- which in turn leading to change in load volt - Zener diode might need to pass lot of current if the Input volt is high compared to output volt |
| --- | --- |
| Series voltage regulator with negative feedback | Link : https://www.youtube.com/watch?v=3n-jcCPBfWI&list=PLXb3r5ny8_1VGu4f8FoKp4ZkdEyd8CF1a&index=5 Issue with the basic series regulator Series regulator with negative feedback - A key element in this ckt is – Error amplifier : it is basically a comparator and compares the output we actually have with the output we want (which uses reference voltage (which can be provided by zener diode)) – we will compare the reference volt to a fraction of load volt (provided via volt divider) - If the voltages are same – nothing to change - But if there is any difference in the voltage – this gets amplified little bit and provides a corrective current that adjusts base emitter voltage accordingly - R1 and R2 forms the voltage divider and we don’t want it drawing large current – so R1 and R2 are kept high - K is the volt divider ration K = R2R2+ R1 - Also, other thing to keep in mind - we don’t want transistor to saturate ( VCE ) – which is basically difference of input and output voltages (VCE > 2v) – so there must be minimum 2 v difference between input and output [ example : if we want 10v as op , we must have >12v input ] Implementation of the circuit - VR = reference volt generated by zener diode (zener operates in the breakdown region and provides reference volt) - The op-Amp is acting as a comparator (also on the same time due to virtual short concept -it tries to match the potential at both of its input (opAmp forces VR = K VL ) - K = R2R2+ R1 - Here, opAmp forms negative feedback (as shown in the image) – this then acts like feedback - Case 1 : Load voltage is too high - If load volt/op volt is high , it makes the volt divider op to be high and the opAmp output lowers down – which eventually makes the output volt/load volt lower (so this feedback loop serves the issue) - Case 2 : Load voltage is too Low - If load volt/op volt is low , it makes the volt divider op to be low and the opAmp output increases – which eventually makes the output volt/load volt to increase (so this feedback loop serves the issue) - Only 2 things to consider when designing this – the resistor should not be too low and also there shall be not high current passing through the zener So adding op-Amp and negative feedback solves the issues we noticed with basic series regulator |
| Re-visiting linear regulators for clear concept | 🔹 1) Shunt Linear Regulator Regulating element is connected in parallel (shunt) with the load. Working idea - It controls current drawn away from the load. - When Vout tends to increase → shunt regulator draws more current → lowers Vout - When Vout tends to decrease → it draws less current → raises Vout Key points Feature; Shunt Regulator. Connection; Parallel to load. Regulates by; Varying current drawn in itself. Efficiency; Low (wastes current). Output control; Limited (only step-down). Simplicity; Very simple. Example - Zener diode regulator → classic shunt type 2) Series Linear Regulator Regulating element is in series with the load. Working idea - It controls voltage drop across itself. - If Vout rises → series regulator increases its dropout → lowers Vout - If Vout falls → reduces dropout → raises Vout Key points Feature; Series Regulator. Connection; In series with load. Regulates by; Adjusting voltage drop through itself. Efficiency; Higher than shunt. Output control; Only step-down. Complexity; More complex than shunt. Examples - Transistor series regulator - Regulators with error amplifier - IC regulators like 7805, LM1117, LM317 → all are series type Internal hierarchy (final picture) Voltage Regulator ├── Linear Regulator │ ├── Shunt Regulator │ └── Series Regulator (IC regulators, LDOs, etc.) └── Switching Regulator ├── Buck ├── Boost ├── Buck–Boost └── others (flyback, SEPIC, etc.) Small note - LDOs (Low Dropout Regulators) are actually a sub-type of series linear regulator → They just use PNP/PMOS instead of NPN to reduce dropout. Ultra-short memory trick Regulator; Location of control element; What it controls. Shunt; Parallel to load; Current. Series; In series with load; Voltage drop. |
| Examples | Link : https://youtu.be/YSjKMSwmhWg?si=hYSb4Ivlaqg4lPaY Shunt regulator - Identified things from problem statement - Vi range Vi = 13.5 ± 1.5 12v and 15v - Rs = 100 ohm - Rz = 5 ohms - RL = 1000 ohms - Vz = 9v - Find - Range of VL - Line regulation - Redrawing the ckt - Here the 9v is shown as battery but it is not – it represents 9v drop due to zener - We need to find relation between input and output (to find the range of load voltage) - Is= IL+ Iz - Vi- VL100=VL- 95+ VL100 - VL=10 Vi - 1800211 - Putting Vi range 12v and 15v - VL=9.100 v (for 12v) - VL=9.242 v (for 15v) - Line regulation (LR) - LR =∆ VL∆ Vi 9.242-9.10015-12 *100% - LR=4.7% - Provided data - Supply volt ( Vi ) = 19.5 ± 4.5 15 to 24 volt - VL = 12v - ILmax = 0.5 Amp - Izmax = 1mA - IopAmp_max = 3mA - VR = 5v For every component we will calculate its value and power rating - RLmin = 120.5 = 24 ohms (if the load resistor value increases – definitely the current is going to reduce so no worries) - Power rating of RL PLmax = VL2RL=6 watt - Now , breakdown current of the zener is 1 mA (so this is the min current flows through zener) - Assuming very little goes to non-inverting side of op-Amp - Thus RS also passes 1 mA current RS= Vi-50.001 - Vi = 15v RS=10 Kohm - Vi = 24v RS=19 Kohm - Now find IS (meanwhile we will also confirm which RS is the practical one (for which any volt range doesn’t show current < 1mA(ideally – for zener to work current through it shall be > min breakdown current) - 24v with 19kohm , 15v with 10 Kohm - will result in IS=1mA - For 15v input and 19 Kohm IS = Vi-5RS = 0.526mA (here – IS is<1mA which makes zener off) - For 24v input and 10 Kohm IS = Vi-5RS = 1.9mA (this is perfect ) - For 24v input and 10 Kohm - IS=1.9mA - For 15v input and 10 Kohm - IS=1 mA - So, the 10kOhm is ideal for us because for lowest to highest volt swing – it keeps the zener on - Power rating of RS PRs = (1.9mA)2*(10000) = 36.1 mW - Now the voltage divider section - The output of volt divider shall be 5v(same as zener) - Thus, applying volt divider formula 5=R2R2+ R1 12v - Here choosing values which is high compared to load resistance - Assuming R2=5k R1=7k - Current drawn by this volt divider/branch I= 1212k=1mA - Power rating of these resistors is in mW - One this we need to keep in mind – this network only draws max (1mA ) which is 0.2% of the max load current of 0.5Amp - Now transistor must handle to max load current (0.5Amp) + divider network current (0.001Amp) - So total current transistor needs to handle will be Itransistor(max)= IE(max)= 0.501 Amp - Base current to transistor is op-Amp’s op current – which is 3mA 0.003Amp - Power rating of transistor Ptransistor=VCE(max) IE(max)→12*0.501=6.012 Watt - Here , if in worst case scenario (if someone shorts output – volt appearing at op =0 and IE(max)= 0.501 Amps Ptransistor=24* 0.501=12Watt (this shows transistor should have power rating of 12Watts ) ( no doubt the current we considered in this case is bound to max current but – in short ckt cases – current is damn high – so over current protection is necessary) - The calculated things which make this ckt work as regulator - RLmin = 24 ohms (power rating > 6W) - RS=10k ohms (power rating > 36.1 mW) - R1=7k ohms (power rating > few mW) - R2=5k ohms(power rating > few mW) - Transistor ( capacity of handling IE(max)= 0.501 Amp (power rating > 12Watt) |
| Overcurrent protection | Link : https://youtu.be/03KeyVS0LAo?si=IHf8mlzETtd76aLi Over current protection ckt - Consider : neglecting the current going in at Rs and in the volt divider network - We will look two situations - Normal operation (when current IE (load current) is within transistor limit ) - Load Short ckt condition (when current IE (load current) becomes large) - Normal Operation (when current IE (load current) is within transistor limit ) - Current coming from unregulated supply takes two paths (towards Q1 (as collector current) and towards RS (this amount is small) - Base current for any transistor is very small (thus – when collector current passes Q1 and becomes emitter current – only a few goes into Q2 – remaining travels towards R3) - Which further branches into volt divider section(this is to be neglected right now) and load section - Now consider voltage across R3 as VR - During normal operation we want transistor Q2 to be off – (for a transistor to be in off condition- we want its VBE<0.7v - So here for Q2 : VBE= VR ( so we want VR to be <0.7v) – for safer side we will keep VR to be <0.6v - Now when the drop across R3 is going to be the highest (when the input supply is the highest) - R3 = 0.6vIL(max) - VR = IL*R3 if IL is less than max current IL(max) : VR is definitely gonna be less than 0.6 (which makes the transistor off) - What means when Q2 is off - No current is passing from Q2’s collector to emitter (consider it as open ckt) - All the current coming out of op-Amp goes into base of Q1 (which is perfect because we want Q1 to be ON under normal condition) - If anything goes wrong at load(such as short ckt which leads to high current) – the VR is going to be higher than 0.6 (which will make Q2 ON) - Load Short ckt condition (when current IE (load current) becomes large) - In this case Q2 becomes ON - Which leads to some base current flowing into Q2 - Q2 starts drawing good amount of current from collector to emitter (which leads to very less current available at Q1’s base) – so as the base current in Q1 reduces which leads to reduction in collector current (same as water tap analogy : the more open the tap is – the more water flows) - We have used transistor Q2 as switch -turning it ON and OFF based on the current flowing through Q1’s emitter (or through R3) Now , revisiting the old example : - We want to protect Q1 in case of load short circuiting - R3 = 0.6vIL(max) = 0.6v0.5Amp=1.2 ohm - We can increase this R3 if we want more drop to turn Q2 on - if the current from Q1 increases beyond 0.5A (Q2 becomes on) and Q1 becomes less conductive leading to reduction in collector current and in turn the emitter current - power rating of R3 PR3=(0.5)2 1.2 =0.3W |
|  | ⚡ Isolated vs Non-Isolated Converters Type; Key Difference; Safety; Components; Example Topologies. Non-Isolated; Input and output share a common electrical ground; Lower safety (no galvanic isolation); No transformer; Buck, Boost, Buck-Boost, SEPIC, Cuk. Isolated; Input and output are electrically isolated (no shared ground); Higher safety (shock protection); Uses transformer; Flyback, Forward, Full-Bridge, Half-Bridge. 💡 Why is a Buck-Boost called Non-Isolated? In a buck-boost converter, energy transfer happens through: - an inductor - a switch (MOSFET) - a diode - and an output capacitor There is no transformer separating the input side and the output side. So the ground (GND) is common → both sides are electrically connected. Therefore → non-isolated topology. 🔍 Where non-isolated converters (like buck-boost) are used - Battery-powered devices (5V → 3.3V or 3.7V → 12V etc.) - Portable electronics - LED drivers - Automotive electronics (12V → 5V or 5V → 12V) They are chosen because: - High efficiency - Cheaper - Smaller size 🔒 When isolated converters are required - Chargers (mobile, laptop) - SMPS inside TVs, PCs - Medical equipment - Industrial power supplies Isolation is needed there for: - Safety from high voltage - Ground noise protection Quick summary Buck-Boost; Output Voltage Range; Isolation. Standard Buck-Boost; Can be higher or lower than input; ❌ No isolation (non-isolated). 🔥 DC-DC Regulators can be of two types: Regulator Type; Non-Isolated; Isolated. Linear; ✔ Almost always non-isolated; ❌ (Isolation is practically impossible in linear topology). Switching; ✔ Yes; ✔ Yes (if transformer is used).. |
| Switching Regulators | 🔋 What is a Switching Regulator? A switching regulator is an electronic power converter that controls output voltage by rapidly switching an electronic component (like a MOSFET) ON and OFF instead of dissipating extra voltage as heat (like linear regulators do). ⚙️ How it Works (Very Basic Idea) - Input voltage enters the circuit. - A high-speed switch (MOSFET) turns ON and OFF many thousands or millions of times per second. - Inductor + capacitor smooth out the resulting pulses into a stable DC output voltage. - Feedback control continuously adjusts the switching duty cycle to maintain the desired voltage. 🌟 Why Switching Regulators Are Popular Feature; Switching Regulator; Linear Regulator. Efficiency; ⭐ Very high (85–95%+); Low (40–60%). Heat; Very low; High. Size; Small (less heat → small heatsink); Larger. EMI Noise; Higher (due to switching); Very low. 🔄 Types of Switching Regulators Type; Function. Buck; Step-down: reduces voltage. Boost; Step-up: increases voltage. Buck-Boost; Can increase or reduce voltage. Flyback / Forward; Used for isolated power supplies. 📌 Where They’re Used You find switching regulators almost everywhere: - Phone and laptop chargers - SMPS power supplies - Electric vehicles - Solar charge controllers - Battery-powered electronics - Graphics cards and CPUs (VRM circuits) Switching regulators achieve high efficiency and low heat by converting power using fast switching techniques + energy-storage components (inductor/capacitor), rather than burning excess power as heat. Other reference video Link : https://www.youtube.com/watch?v=CPvA-uS0dNY&list=WL&index=2 |
| Buck Converter / Buck regulator | Link : https://www.youtube.com/watch?v=O_rhsgEKpTM https://youtu.be/Q7cTuZIH8IA?si=V792LKbHquSHgqB0 Key function - Ci = Input capacitor → Provides a reservoir of energy and reduces input ripple. - Q1 = The switching MOSFET → Turns ON and OFF at high frequency (tens of kHz to MHz). → Controlled by PWM. (– the time on is the mosfet on and the time off is mosfet off) (The inductor is the component mainly used for its property that it doesn’t allow to change the current immediately) - L1 — Inductor Purpose: Store energy and smooth current (L1 keeps current continuous.) - When Q1 turns ON, the inductor stores energy (current ramps up). - When Q1 turns OFF, the inductor releases energy to the output (current keeps flowing). - The inductor prevents current from dropping to zero between switching cycles, so the load gets almost continuous power. - Co — Output Capacitor Purpose: Reduce output voltage ripple (Co keeps voltage smooth.) - Acts like a buffer/reservoir at the output. - Supplies current instantly when the load needs it. - Reduces the voltage ripple caused by switching. - Without Co, the output voltage would look like a sharp sawtooth instead of a smooth DC. - D1 — Freewheel (Flyback) Diode Purpose: Provide a path for inductor current when Q1 is OFF (D1 gives current a path when the switch turns OFF.) - Inductor current must always flow. When Q1 switches OFF: - D1 becomes forward-biased - Inductor current flows through D1 into the load and Co - This prevents a high-voltage spike that would otherwise damage Q1. [Q1 chops current → L1 smooths current → Co smooths voltage → D1 keeps current flowing when Q1 is OFF. ] How does it work There are two cases - MOSFET is ON - MOSFET is OFF - MOSFET is ON - When the MOSFET Q1 is ON - Current starts flowing through L1 Co and Load gnd Vin Q1 L1 … (loop is completed) - (current through L1 – in which L1 opposes the change of current – so the graph of current through L1 will be a ramp) - MOSFET is OFF - When the MOSFET Q1 is OFF - L1 doesn’t like the change in current (so its magnetic field collapses and tries to maintain the current ) - Current starts flowing through L1 Co and Load D1 L1 …(loop is completed) - (current through L1 – in which L1 opposes the change of current – so the graph of current through L1 will be a ramp) - So now ON and OFF MOSFET periodically to control the avg current flowing through the inductor (BECAUSE THE INDUCTOR CURRENT IS THE LOAD CURRENT) - Voltage drop across load = I * R = current flowing through the load(or current through inductor) * load resistance Waveforms based on working of Buck converter - Note : Here Vout shall be almost constant in Toff – but not shown here (wrong in the video) Here, after calculation Vout=Duty cycle* Vs - Duty cycle is always < 1 - Thus, this equation shows the output will be always less than Vs - which is basically what we want in buck converter (you can set the duty cycle input to mosfet to achieve required output) 🔥 Two types of buck converters There are two major ways to provide the return path for inductor current when the high-side switch (Q1) is OFF: ① non-synchronous buck converter Uses a diode (D1) for the current path during OFF time. ② Synchronous buck converter Uses a second MOSFET instead of a diode to provide the current path. 🟥 Non-Synchronous Buck Converter (your diagram) Q1 (switch) + inductor + capacitor + diode - Q1 turns ON → energy flows from input to inductor - Q1 turns OFF → diode D1 conducts, letting inductor current flow ✔ Simpler ✔ Cheaper ❌ Less efficient (because diode drop wastes power: ~0.5–0.7 V loss) 🟦 Synchronous Buck Converter Q1 (high-side MOSFET) + Q2 (low-side MOSFET) + inductor + capacitor - Q1 ON → energy flows to inductor - Q1 OFF → Q2 (MOSFET) turns ON instead of a diode ⚡ Why this is better: - MOSFET has very low Rds(on) → drop maybe only 10–50 mV instead of 0.7 V of a diode - Great efficiency — especially for heavy loads and low voltage outputs ✔ Higher efficiency ✔ Better thermal performance ❌ More complex (requires timing so Q1 and Q2 never turn ON together — shoot-through protection) 🔁 Simple analogy Type; Analogy. Non-synchronous buck; Water allowed back only through a one-way spring valve (diode). Synchronous buck; Water returns through a second controlled gate (MOSFET). ⚡ When each is used Non-Synchronous; Synchronous. Low cost circuits; Performance / high efficiency. Low to medium current; Medium to high current. > Output 5 V; Perfect for low-voltage outputs (e.g., 1–3.3 V). Simple design; Needs good gate-drive timing. Example ICs: - LM2596 → non-synchronous - MP2307, TPS5430, many CPU VRMs → Synchronous 🔑 Final one-line summary Non-synchronous buck uses a diode for the current return path; synchronous buck uses a MOSFET instead of a diode for higher efficiency.. |
| Boost Converter / Boost regulator | Link : https://www.youtube.com/watch?v=I0ZbC7uCe9A Two conditions - MOSFET switch is ON - MOSFET switch is OFF Note : here the ‘S’ shown is considered as a switch (which is basically MOSFET switch operated by PWM – ON time of the PWM pulse is the ON period of MOSFET and OFF time of the PWM pulse is the OFF time of MOSFET MOSFET is ON - Here when the switch is on- it provides a very low resistance path - Hence the pink loop shown in image is where the current flows – charging Inductor (magnetic field across it builds – you see a ramp like waveform for current - As the switch path is parallel to load (and no drop occurs across it – meaning 0 volt drop – thus Vout=0 ) - Inductor voltage keeps increasing in this period MOSFET is OFF - As the switch is off (open)– the path : Vs Inductor Diode Load and Capacitor Vs - Inductor hates change in current so it tries to keep it same – to do so it collapses its magnetic field and provides reverse volt to keep the current same - Now the voltage seen by Load is Vout= Vs+ VL - Diode is there for allowing current in one direction - Capacitor is for smoothing the output ripple - Here - Vout= Vs+ VL - Vout= Vs+ L dIdt - Vavg= ( Vs+ VL )* ToffTON+ Toff - This Vavg will be always > Vs (which is perfect for a boost converter/regulator) |
| Buck Boost Converter | Link : https://www.youtube.com/watch?v=YiYQjdARZ7I&t=8s BUCK-BOOST converter = Inverting Regulator Function is explained in 3 parts - MODE – 1 : MOSFET is ON - MODE – 2 : MOSFET is OFF - MODE – 3 : MOSFET is OFF MODE-1 : MOSFET is ON - Loop highlighted with Red marker - As shown – current flowing path [ voltage source mosfet and C1 inductor voltage source] - The inductor starts charging - Diode is reverser bias (so no current flows there ) - Voltage across inductor is equal to source voltage MODE-2 : MOSFET is OFF - Loop highlighted with orange marker - MOSFET switch is off - Now inductor starts discharging (inductor magnetic field collapses trying to maintain const current – volt polarity reverses) - Current direction : [ inductor capacitor and load diode inductor ] - Here C2 charges by lower plate being positive - Vout will be reverse polarity VL voltage MODE – 3 : Inductor is left with no energy (its magnetic field is totally empty) - Loop highlighted with blue marker - Here MOSFET is off , energy of inductor is all used. - Now as inductor is left with no energy - Capacitor is the only voltage reservoir available - Now capacitor discharges as shown by blue markings - However now , the load gets the reverse polarity volt as output (this is the reason why it is also called as Inverting regulator) [ the volt available has -ve polarity compared to source volt ] - Vout will be reverse polarity VC voltage - VL = voltage across inductor - IL = current in inductor - IT = current in MOSFET - ID = current in diode - VL (inductor voltage) - mode 1 (switch is ON) – voltage across inductor will be risen to source volt V - mode 2 (switch is OFF) – voltage across inductor will reverse its polarity to keep the current from changing - mode 3 (switch is OFF) – voltage across inductor is now 0 as all of its saved energy is now emptied - IL (inductor current) - mode 1 (switch is ON) – current across inductor will be risen to max (this is ramp because inductor opposes the changes in current) - mode 2 (switch is OFF) – current across inductor will be lower to 0 (this is ramp because inductor opposes the changes in current) - mode 3 (switch is OFF) – current across inductor is now 0 as all of its saved energy is now emptied - IT (MOSFET current) - mode 1 (switch is ON) – current increases (seen as ramp – because inductor opposes change in current) - mode 2 (switch is OFF) – current is 0 (as MOSFET is off – no current flows through it) - mode 3 (switch is OFF) – current is 0 (as MOSFET is off – no current flows through it) - ID (current in diode) - mode 1 (switch is ON) – current doesn’t flow through diode - mode 2 (switch is OFF) – current is same as inductor current - mode 3 (switch is OFF) – current is 0 (as all of inductor’s saved energy is now emptied) NOTES - IN mode-3 ckt is acting as buck regulator Vo=V D1-D (here D=duty cycle – value varies from 0 to 1) - MODE 1 is MOSFET ON period ( Ton ) - MODE 2 and MODE 3 is basically MOSFET OFF period ( ToFF ) - So, if we keep Ton time higher – we get boost converter and if Ton time lower , we get buck converter Vo=V D1-D - Here , - If Duty cycle is 0.25 we get Vo= V3 (buck converter) - And if Duty cycle is 0.75 we get Vo= 3V (boost converter) - So, by keeping Duty cycle > 0.5 (we get boost converter) and by keeping duty cycle < 0.5 we get buck converter - And if we keep duty cycle =0.5 we get same volt as input (which is useless) |
|  | 🔥 Q1: What is Thermal Shutdown in a regulator? Every regulator (linear or switching) generates heat during operation. If the temperature rises too high due to: - high load current - high input–output voltage difference (linear regulator case) - bad heatsinking - high ambient temperature - short circuit …then the internal temperature can exceed a safe limit. To prevent permanent damage, regulators have a feature called Thermal Shutdown (TSD). 🔍 How it works - An internal temperature sensor monitors the die temperature. - When temperature crosses the threshold (typically 150°C to 175°C): ➜ regulator automatically turns OFF (cuts output) - When temperature falls back to safe region (usually 130°C to 140°C): ➜ regulator turns ON again automatically It is a self-protection mechanism to prevent: - silicon damage - overheating - fire risk ⚠️ Key point Thermal shutdown protects the regulator itself — but it is not a substitute for proper heatsinking. 🔥 Q2: You know over-current protection in linear regulator using a transistor — is it same for switching regulator? Short answer: Concept is same — goal is to limit current — but the method is completely different. ⚡ In Linear Regulators Over-current protection is typically done using: - current sensing resistor - sense transistor - foldback / constant current limiting Example (LM7805): If current > limit: - pass transistor is forced into limiting - output voltage drops down - power dissipation occurs in the pass transistor - heats up → thermal shutdown may trigger So in linear regulator: overcurrent + high dissipation → big heating → thermal shutdown may activate. ⚡ In Switching Regulators Here current limiting is implemented differently because switching converters don’t drop excess voltage as heat. Methods used: ✓ Peak current mode limiting - Controller senses inductor current - If current peak exceeds limit → turns OFF MOSFET immediately ✓ Cycle-by-cycle current limit - Every switching cycle checks current - Prevents MOSFET from entering saturation ✓ Short-circuit protection - If output is shorted → switching frequency reduced or latched-off ✓ Hiccup mode - Regulator tries to start repeatedly like: ON → trip → OFF (cool) → retry - Prevents overheating ⚠️ Big difference Feature; Linear regulator; Switching regulator. During overcurrent; Pass element dissipates heat; MOSFET turns off immediately. Voltage during limit; Drops; Pulses / shutdown / hiccup. Efficiency; Very low; High. Risk of burn; High; Low. 🧠 Final Summary Q1: Thermal shutdown Over-temperature protection that turns the regulator OFF when internal temperature exceeds a safe limit and turns it ON when cooled. Q2: Over-current protection Regulator type; Concept; Method. Linear; Limits current; Sense resistor + limiting transistor. Switching; Limits current; Peak current sensing + cycle-by-cycle cutoff / hiccup / shutdown. Both protect against over-current, but implementation is completely different.. |
|  | A) LINEAR VOLTAGE REGULATOR – Major Components & Their Functions (Example: 7805 / LM1117 / LM317 type) Component; Function. Pass Element (Series Transistor – BJT or MOSFET); Adjusts resistance to keep output voltage constant; acts like a variable resistor between input & output. Voltage Reference; Provides a precise fixed reference voltage internally (e.g., 1.25V). Error Amplifier (Op-amp); Compares output voltage to reference voltage and drives the pass transistor. Feedback Network (Resistors / internal divider); Samples the output voltage and feeds it to error amplifier. Protection Circuits; • Over-current limit (sense transistor) • Foldback current limit • Thermal shutdown. Startup / Bias Circuit; Provides internal power to the reference and error amplifier. 🔑 How they work together (simple flow) Output voltage (sampled) → compared with reference → error amplifier output → controls pass transistor → adjusts voltage until regulation achieved. B) SWITCHING VOLTAGE REGULATOR – Major Components & Their Functions (Example: buck / boost / buck-boost converter) Component; Function. Power Switch (MOSFET / internal switch); Turns ON/OFF rapidly to control energy flow. Diode (or synchronous MOSFET); Provides a path when the main switch is OFF (free-wheeling). Inductor; Stores and releases energy; smooths current. Output Capacitor; Smooths voltage ripple and supplies load during switch transitions. Controller / PWM IC; Generates switching signal (PWM) to maintain output voltage. Error Amplifier + Voltage Reference; Compares output voltage with reference and adjusts PWM duty cycle. Feedback Network (resistor divider); Samples the output voltage and feeds it back to the controller. Current Sensing Circuit; Detects inductor/MOSFET current for cycle-by-cycle current limit and stability. Protection Circuits; Over-current, short-circuit, over-voltage, thermal shutdown. Driver (Gate Driver); Amplifies PWM from controller to drive power MOSFET strongly. 🔑 How they work together (simple flow) Output voltage (sampled) → compared with reference → error amplifier → duty cycle adjusted → drive MOSFET switching → inductor + capacitor filter output. ⚡ One-shot comparison to remember forever Feature; Linear Regulator; Switching Regulator. Main control element; Pass transistor (variable resistor); MOSFET switch (ON/OFF). Energy conversion; Dissipates excess power as heat; Stores & transfers energy with LC. Efficiency; Low at high drop; Very high. Output Ripple; Very clean; Needs filtering. Complexity; Simple; Complex. Components count; Low; High. 🧠 Core takeaway Linear regulator controls voltage by varying resistance. Switching regulator controls voltage by varying duty cycle of switching.. Hiccup mode in switching regulators means Periodic restart after fault. This is a protection mechanism where the regulator shuts down when a fault is detected (like overcurrent or short circuit) and then attempts to restart automatically after a brief delay or period [1]. If the fault persists, it shuts down again, creating a "hiccup" cycle until the fault is removed [1]. This method significantly reduces power dissipation during a fault condition compared to continuous operation or permanent shutdown, thus protecting the regulator from damage. Inside a System on a Chip (SoC), voltage regulation is usually provided by On-chip LDOs / PMICs (Low-Dropout Regulators / Power Management Integrated Circuits) SoCs often require multiple different supply voltages for various components like the CPU core, memory, and I/O, which differ from the single external voltage supplied to the chip [1]. On-chip LDOs and PMICs are integrated directly into the chip design to efficiently manage and regulate these internal voltages as needed. Q27. Unstable VDD mainly affects digital circuits by causing: Timing violations Unstable supply voltage (VDD) affects the delay of logic gates. If the voltage drops (voltage droop), gates slow down, which can cause signals to arrive later than expected, leading to timing violations (setup and hold time issues) and potential functional failures Q28. Front-end VLSI engineers mostly care about regulators because of: Power integrity and timing While front-end engineers primarily focus on RTL design, synthesis, and functional verification, they must consider the physical constraints and effects that impact the final chip. Voltage regulators directly influence power integrity (PI) by ensuring a stable VDD. Poor power integrity leads to voltage fluctuations (noise and droop), which, as noted above, directly cause timing violations that must be accounted for during static timing analysis (STA) and design sign-off. |
| Developing circuits in LTspice [Linear Regulators] | Zener shunt regulator - Build circuit like this - To bring the zener – choose normal diode right click pick new diode select TDZV9_1 (9.1 zener diode from Rohm) - Test1 : check weather this ckt works as a 9.1v regulator - Checkpoint : output shall be 9.1v - [ Go to : simulate Configure Analysis DC op pnt Syntax : .op ] or [ spice directive (symbol with .t) write ‘.op’ and click ok ] - Now click Run - You will see window opening as shown below - Check Vout – it should be zener break down volt (i.e. 9.1v) - Test2 : check changes in output over change in input [line regulation] - Checkpoint : Vout shall be within 9.00 to 9.2v - [ Go to : simulate Configure Analysis DC sweep Enter as shown in image ] [ this means change V1 from 10v to 15v , with step size of 1v] - or [ spice directive (symbol with .t) write ‘.dc V1 10 15 1’ and click ok ] - now run simulation a blank graph will open - click on Vout (u will see a probe like thing) - this will plot Vin vs Vout graph as shown below [here – u will see Vout varying very little – from 9.08 to 9.11v] [ you can try with other values] - Now, let’s find out line regulation [change in output due to change in input] - ΔVin = 15 − 10 = 5 V - ΔVout = 9.11 − 9.08 = 0.03 V - Line regulation (in V/V): Line Regulation=ΔVoutΔVin=0.035=0.006 V/V - As a percentage of output (~9 V = nominal output): - Line regulation in % = ΔVoutΔVout(nominal) ×100% 0.039≈0.0033=0.33% - [So, line regulation is roughly 0.33% change in Vout for a 50% change in Vin (10 → 15 V). That’s very good for a simple zener shunt circuit.] - Test Case 3 : check output volt for changes in load resistor [load regulation] - First make volt = 15vDC - Now right click on load resistor and enter ‘{RL}’ instead of value (Now LTspice knows: “this resistor’s value depends on a parameter named RL”) - Ckt will look like this - We will use spice directive(symbol with .t) - Add this ‘.param RL 1k’ and this ‘.step param RL 500 2000 500’ below it - This means - RL will go from 500Ω 1000Ω 1500Ω 2000Ω (Step size 500) - After placing it on screen if you write click it [‘.step param RL 500 2000 500’ ] (it will show window like this) - If you right click ‘.param RL 1k’ – this will open below shown window - Meanwhile we also don’t want input sweep to take place this time because we are checking the function of load regulation (so we need to comment ‘.dc V1 10 15 1’ by adding a semicolon Infront of it ‘; .dc V1 10 15 1’ (as shown in the image) click ok - Now you should see something like this - Now run the simulation (you might have to run few times before actually the blank graph opens up) - Click on Vout (you will see probe like symbol) - You will see (Vout vs Rload graph) - The X axis -> Rload (varying from 500 ohm to 2000 ohm) - And for this varying load output varies from 9.1145 to 9.1186 Rload; Vout. 500 Ω (heavy load → more current); 9.1145 V. 2000 Ω (light load → less current); 9.1186 V. ΔVout = 9.1186−9.1145 = 0.0041 V ≈ 4.1 mV (That means when the load resistance quadrupled, the output voltage changed only by 4 mV.) - Why did Vout increase when RL increased? - Because , - Higher RL → smaller load current - Less current is demanded from the series resistor - More current flows into the zener diode instead - Zener holds the voltage more strongly due to higher Iz - So Vout slightly increases RL; Load current; Zener current; Regulation strength. Low (500Ω); High; Low; Slightly weaker. High (2000Ω); Low; High; Slightly stronger. - Load regulation - Where: - VNL≈9.1186 V(no/light load at RL=2000Ω) - VFL≈9.1145 V(full/heavy load at RL=500Ω) load regulation= VNL- VFLVFL ×100% - Load regulation ≈ 0.045% (excellent) Series Regulator wo over current protection - Test case 1 : Vout should be Vz – Vbe = 5.6 – 0.7 = 4.9v - Here if input is 12v , output shall be fixed to Vz-Vbe = 5.6-0.7 = 4.9v - Test case 2: check line regulation : check output when the input is changing - The output changes from 4.895 to 4.903v - Now, let’s find out line regulation [change in output due to change in input] - ΔVin = 15 − 10 = 5 V - ΔVout = 4.895 – 4.903 = 0.008 V - Line regulation (in V/V): Line Regulation=ΔVoutΔVin=0.0085=0.0016 V/V Line regulation in % = ΔVoutΔVout(nominal) ×100% 0.0085 = 0.16% - Test case 3: check load regulation : check output when the load is changing - Load regulation - Where: - VNL≈4.9375V(no/light load at RL=2000Ω) - VFL≈4.8994 V(full/heavy load at RL=500Ω) load regulation= VNL- VFLVFL ×100% - Load regulation ≈ 0.77 % (excellent) - Load regulation is the perfect for a linear regulator such as this |
| Switching regulators | BUCK converter / Buck Regulator - Test case 1 : Vout should be Vout​=D×Vin​ [D=duty cycle] - Period = 10 µs - On-time = 4.2 µs - Duty Cycle = 4.2 / 10 = 0.42 → 42% - Vout​ = 0.42×12 ≈ 5.04 V - Add this on scr as spice directive “.model Sbuck SW(Ron=0.01 Roff=1e6 Vt=2 Vh=0.5)” - And add configuration analysis of “.tran 0 5m 0 100n” [stop time = 5ms and time step = 100ns] - Run simulation - you will see shown graph (in starting it shows fluctuations) - That fluctuation at the beginning of buck-converter output graph is simply the LC resonance (ringing) and settling of the output filter when the PWM first starts switching. (if we increase the initial delay of applying pulse to switch by >3ms(settling period) – we will see steady state behaviour of ckt) - The actual output we are getting is 4.56v instead of calculated 5.04v - Why?? - Voltage drop across diode (≈0.4–0.5 V) - Inductor ripple - Switch drop - Load dynamics - Thus, we get 5.04−0.45≈4.59 V as output - Test case 2 : Line regulation check - Make value of V1 = “{Vin}” - add spice directive “.param Vin = 12” and “.step param Vin 10 15 1” to check line regulation [ Note : Green -> Vin = 10v Blue -> Vin = 11v Red -> Vin = 12v Teal -> Vin = 13v Purple -> Vin = 14v Grey -> Vin = 15v ] - for Vin = 10 output is 3.73v - for Vin = 15 output is 5.84v - line regulation - Line Regulation=ΔVoutΔVin=2.115=0.422 V/V - Line regulation in percentage - Line regulation in % = ΔVoutΔVout(nominal) ×100% - Nominal output = 4.5 Line Regulation (%)= 2.114.5 ×100%=46.8% - Test case 3 : load regulation check - Update R1 value to be “{RL}” - Add spice directive “.step param RL 5 55 10” to check load regulation - You will see below shown graph [ Note : Green -> RL = 5 ohm 4.58v Blue -> RL = 15 ohm Red -> RL = 25 ohm Teal -> RL = 35 ohm Purple -> RL = 45 ohm Grey -> RL = 55 ohm 5.87v ] - Load regulation - Where: - VNL≈5.87 V(no/light load at RL=55Ω) - VFL≈4.58 V(full/heavy load at RL=5Ω) load regulation= VNL- VFLVFL ×100% - Load regulation ≈ 28.16% Boost converter / Boost Regulator Vout = Vin1-D (ideal output volt) - Duty cycle = 0.42 - Vout = 20.68v - Practical vout = 19.91 and load current = 397.41mA BUCK - BOOST converter - output of this inverting buck-boost converter is inverted - when D < 0.5 (ckt behaves as buck converter) and when D > 0.5 (ckt behaves as boost converter) - Vout= D1-D Vin - If we keep the duty cycle = 0.66 [ Boost converter behaviour] - Vout = -23.29v - Actual Vout = -22.56v (almost 0.7v difference due to diode loss) - If we keep the duty cycle = 0.42 [Buck behaviour] - Vout = -8.68v - Actual Vout = -7.97v (almost 0.7v difference due to diode loss) |