| Formula | Explanation |
| --- | --- |
| C = QV | C = capacitance Q = charge V = voltage |
| Energy stored in capacitor | E = 12 C V2 |
| C = A Ɛd | A = Area of the plates Ɛ = permittivity of dielectric ( Ɛ = Ɛ0 * Ɛr) - Ɛ0 = Absolute permittivity - Ɛr = relative permittivity (or dielectric constant of the material) d = distance between the plates / thickness of the plates |
| Leakage current (as a spec in capacitor) or insulation resistance (also sometimes called as parallel resistance) | In some specs of capacitor where leakage current in µA to mA range it is labelled as leakage current and in some capacitor, where is low it is labelled as insulation resistance Rin or Rp= VIleak (results in Giga-mega range) |
| ESR (equivalent series resistance ) ESL (equivalent series Inductance) | Whenever we mention use of capacitor (we consider its nominal value -however in actual scenario it contains some series resistance and series inductance and some parallel insulation resistance) ESR = it’s a small resistance present inside the capacitor due to its leads, plates, and electrolyte ESL = ESL stands for Equivalent Series Inductance, and it represents the tiny inductance present inside a real-world capacitor. Capacitor behaviour is freq dependent At low frequency – (0 Hz = DC) and freq < f resonant capacitor’s capacitance is only responsible as impedance At f= fresonant – ESR dominates (capacitor impedance = ESR (capacitance and parasitic inductance cancels out each other) And at high freq (f>fresonant) – ESL dominates (capacitor impedance = ESR + ESL) In nutshell: (Capacitor behaviour is frequency-dependent ) - At low frequencies (including DC = 0 Hz): The capacitive reactance dominates (Xc) — the capacitor behaves like a capacitor. - At the resonant frequency: The effects of capacitance and ESL cancel each other: (so we’re left with ESR only) (The total impedance is at its minimum, equal to just ESR) - At frequencies above resonance: ESL dominates — the capacitor starts acting like an inductor. Capacitor behaviour = frequency driven ✅ Low freq → Capacitance dominates capacitor actually behaves like capacitor ✅ f = Resonant freq → ESR dominates (minimum impedance) ✅ High freq → ESL dominates → cap behaves inductive Sometimes manufacturers mention dissipation factor instead of ESR Real Word capacitor Application Example. 1. Power Supply Filtering (Decoupling Capacitors) Used in: Microcontrollers, CPUs, FPGAs, VLSI cores 🔧 The Problem: - Digital ICs switch ON/OFF millions of times per second. - This causes fast current spikes on the power line → creates voltage noise/ripple. ⚡ What You Need: - A capacitor that quickly supplies or absorbs charge during those spikes. - Low ESR → so it doesn’t waste energy or drop voltage - Low ESL → so it can respond to very fast changes (MHz–GHz) 💡 Real-World Fix: - Place a ceramic SMD capacitor (e.g., 0.1µF) very close to IC pin - It has very low ESR + ESL → perfect for high-frequency decoupling. 🛠️ 2. Analog Filters (RC Filters) Used in: Audio circuits, sensor signal processing, anti-aliasing 🔧 The Problem: - You want to pass low-frequency signals and block noise - You design an RC low-pass filter ⚡ Why ESR/ESL matter: - If ESR is too high → the filter is less sharp - If ESL is high → the filter fails at higher frequencies, or lets spikes through 💡 Example: A bad-quality capacitor in an audio filter might allow high-pitched noise or hum to pass through.. 📡 3. RF Circuits / High-Speed Communication Used in: WiFi, Bluetooth, RF amplifiers, Ethernet 🔧 The Problem: - Signals in these circuits are in the MHz to GHz range - Any capacitor with ESL acts like an antenna or inductor ⚡ Why ESL is dangerous: - At high freq, ESL causes the cap to stop acting like a cap - It reflects signals or distorts waveform 💡 Fix: - Use special low-inductance ceramic caps - Keep capacitor physically very close to the signal line - Use multiple caps (0.1µF, 0.01µF, etc.) in parallel → better across frequency range. 🔋 4. Bulk Power Storage / Battery Backup Used in: Routers, clocks, EEPROM hold-up 🔧 The Problem: - You need to store charge for longer durations (slow discharge) ⚡ Why Rp (leakage) matters: - If Rp is low, the capacitor leaks slowly and drains faster - Your backup time becomes shorter 💡 Solution: - Use capacitors with high Rp (low leakage) - Tantalum or electrolytic caps are common here. 💣 5. Switching Regulators (SMPS) Used in: Laptop chargers, phone chargers, motor drives ⚡ Problem: - These circuits switch power at high speed (50kHz to 1MHz) - Capacitor must filter output cleanly, else it causes ripple, heat, noise 🔧 Why ESR matters: - If ESR is high, ripple increases and components heat up - You may even get oscillation in the circuit 💡 Solution: - Use Low ESR electrolytic capacitors or polymer caps - ESR is often mentioned clearly in datasheets for SMPS designs. |
| Dissipation factor ( DF) | The DF of a capacitor is the measure of the energy lost as heat during its operation (it should be as low as possible) DF = ESR¦ Xc ¦ Xc = 12 π f c [ f = frequency in Hz , c = capacitance in farad ] |
| Volumetric Efficiency (VE) | It tells you how much capacitance (in µF) a capacitor can provide per unit volume (in cm³ or mm³). 🧠 Why It Matters: In modern electronics: - Space is precious (think smartphones, wearables, VLSI chips) - So, designers want maximum capacitance in minimum space - Hence, high volumetric efficiency = better design choice! 🔎 Example: Let’s say you have: - A ceramic capacitor: 1 µF in a 1 mm³ package → VE = 1 µF/mm³ - An electrolytic capacitor: 10 µF but takes 20 mm³ → VE = 0.5 µF/mm³ Even though electrolytic has more capacitance, ceramic is more efficient per volume. |
| Capacitors in parallel and in series | Cs= C1+ C2+ C3+… (in parallel) – capacitor in parallel increases capacitance (think of it as capacitor plate size increases and dielectric thickness is same) 1Cp= 1C1+ 1C2+ 1C3+… (in series) – capacitor in series decreases capacitance below the lowest capacitance in series (think of it as capacitor in series increases the dielectric material) [the below given example is for dc ] Note : - Parallel capacitor stores same charge - Ex. C1 = 10uf , C2 = 220uf , C3 = 100uf ctot = 330uf = 0.00033F - Total charge (Q) = tot capacitance * volt = 0.00033 * 9v = 0.00297 coulombs - Charge across capacitors Q1 + Q2 + Q3 = Q (here charge is diff for diff capacitors) - Capacitor in series stores charge differently - Ex. C1 = 10uf , C2 = 220uf ctot = 1 / (1/c1 + 1/c2) = 9.56uf (less than the lowest capacitor(c1)) - Ex. C1 = 10uf , C2 = 220uf , C3 = 100uF ctot = 1 / (1/c1 + 1/c2 + 1/c3) = 8.73uf (less than the lowest capacitor(c1)) - Total charge (Q) = tot capacitance * volt = 0.0000873 * 9v = 0.0000785 coulombs - Charge across capacitor c1 = Q , c2 =Q and c3 =Q (as the current is same in series so does the charge across capacitor - Hence max voltage v1 in capacitor c1 Q/C = 0.0000785/0.00001 = 7.857 v - Hence max voltage v2 in capacitor c2 Q/C = 0.0000785/0.00022 = 0.357 v - Hence max voltage v3 in capacitor c3 Q/C = 0.0000785/0.0001 = 0.786 v - Here v1 + v2 + v3 = V = 9v = total volt of battery |
| Capacitor charging time | Here 5 sections = 5 time constant (τ) is needed to achieve 99.3% of the volt applied Time constant (τ) = resistance (ohm) * capacity(farad) = 10000 * 0.0001 = 1 Hence time taken for this capacitor to charge upto 99.3% is 5* 1 sec = 5 sec So After 1st time constant voltage reaches 63.2% After 2nd time constant voltage reaches 86.5% After 3rd time constant voltage reaches 95.0% After 4th time constant voltage reaches 98.2% After 5th time constant voltage reaches 99.3% Note : as the voltage in capacitor increases the current decreases (as the capacitor blocks DC after being fully charges. So while charging it reduces the current flow by accumulating the charge) |
| Capacitor dis-charging time |  |
| Capacitor , inductor and resistor behaviour with AC | - DC current/voltage is constant - AC current/voltage is sinusoidal (alternating with time) - Thus in DC there being no phase difference between current and voltage (think of it as there is no physical lag – ex voltage changes suddenly-current will change suddenly and vice versa) - In AC – - Resistor – does not do anything with current or voltage that makes them lag or lead each other - Capacitor opposes changes in voltage — when voltage tries to change suddenly, the capacitor charges/discharges to smooth it out, causing voltage to lag behind current. Hence, current leads voltage by 90°, which means a −90° phase angle. - Inductor opposes changes in current — when current tries to change suddenly, the inductor develops a voltage that resists that change. As a result, current lags voltage by 90°, giving a +90° phase angle. |
| Capacitor power in AC circuits | - In DC voltage and current has no phase angle between them - In AC voltage and current are sinusoidal - Power here in terms of AC has two components (Active power (that is been consumed or used) + reactive power (that is been consumed but released also (so net power is 0)) (ex. capacitor saves energy in form of charge and then releases it when needed –thus net power is 0) - S = P(active power) + Q(reactive power) = S=P+jQ (jQ is imaginary as it gets consumed and released so net power is 0) Real (Active) Power, P - Power that actually does useful work - Converts into heat, light, motion, etc. - Unit: Watt (W) - Formula: P = VI cos Ø Reactive Power, Q - Power that is stored and returned to the source - No actual energy is consumed permanently - Unit: VAR (Volt-Amp Reactive) - Formula: Q = VI sin Ø - For resistor S = P + Q = VI cos(0) + VI sin(0) = VI (1) + VI (0) = VI [only real part exists for resistor] - For Capacitor S = P + Q = VI cos(-90) + VI sin(-90) = VI (0) + VI (-1) = -VI [only imag part exists for capacitor as it consumes and releases power so net power is 0] - For inductor S = P + Q = VI cos(90) + VI sin(90) = VI (0) + VI (1) = VI [only imag part exists for inductor as it consumes and releases power so net power is 0] So, for capacitor S = P + Q = VI cos(-90) + VI sin(-90) = VI (0) + VI (-1) = -VI |
| Lag / Lead in phase | Note : lag and lead only exist in AC as due to existence of components like capacitor and inductor (bottom line -- due to their reactances) - In capacitor current leads voltage by 90 degrees (-90degree phase shift) - In inductor voltage leads current by 90 degrees (+90degree phase shift) But what is exactly lag and lead For a sine wave: - One full cycle = 360° - 90° = 1/4 cycle So, if you're at 50 Hz: - One cycle time period T = 1f = 150 = 0.002 sec = 20 ms - 90° lag 14 20 ms = 5 ms delay caused by reactances - Voltage hits its peak 5 milliseconds after current does (in a capacitor at 50 Hz -due to Xc component). - Voltage hits its peak 5 milliseconds before current does (in an Inductor at 50 Hz -due to XL component). |
| Reactance | Reactance is the opposition a component offers to AC current, due to its ability to store and release energy. Think of it like: - Resistance opposes current in general (DC or AC) - Reactance opposes current only when current is changing (i.e., in AC) Reactance is similar to resistance, but: - It doesn't consume energy (no power loss) - It stores and releases energy (in electric or magnetic field) - It is measured in ohms (Ω) — same unit as resistance! Capacitor resonant freq fres= 12 π LC There are Two Types of Reactance: 1. Capacitive Reactance (Xc) - Opposes low-frequency AC signals more - Formula: XC= 12 π f C - f = frequency in Hz - C = capacitance in farads - At low frequency → XC is high (blocks AC) - At high frequency → XC ​ is low (allows AC) 2. Inductive Reactance (XL) - Opposes high-frequency AC signals more - Formula: XL= 2 π f L - f = frequency in Hz - L = inductance in henrys - At low frequency → XL is low (allows AC) - At high frequency → XL​ is high (blocks AC) ⚙️ Total Opposition in AC = Impedance Reactance combines with resistance to form impedance: Z = R + j X - Z = impedance (total opposition to AC) - X=XL ​− Xc - j = imaginary unit (shows phase shift) |
| Resonant Frequency | The resonant frequency is the frequency at which a circuit containing a capacitor (C) and an inductor (L) naturally oscillates. At this frequency: XL ​= Xc As a result: - Their reactances cancel each other out - The total reactance becomes zero - The circuit behaves like it has only resistance - The impedance is minimum (in series) or maximum (in parallel) For a standard LC circuit (without resistance): fo= 12 π L C - fo = resonant frequency (Hz) - L = inductance (henry, H) - C = capacitance (farad, F) ▶ In a Series LC Circuit: - XL ​= Xc​ → net reactance = 0 - Impedance is minimum - Acts like a short circuit for AC at that frequency - Allows maximum current to flow - Used in tuned amplifiers, bandpass filters, antenna matching ▶ In a Parallel LC Circuit: - XL ​= Xc​ → they create a very high impedance - Total current drawn from the source is minimum - Circuit acts like an open circuit to AC at that frequency - Used in oscillators, band-stop filters, tank circuits |
| Series LC and parallel LC | For AC, remember: - Capacitive Reactance: - Decreases with frequency - Xc= 12 π f C - Inductive Reactance: - Increases with frequency - XL= 2 π f L Resonance Condition At resonance: XL ​= Xc => net reactance=XL ​- Xc ​=0 1️⃣ Series LC Circuit at Resonance Circuit: AC Source → L → C → back to source What happens: - L and C are in series - At resonance: XL ​= Xc => net reactance=XL ​- Xc ​=0 - So total reactance = 0, and if resistor is negligible, - Z = R + j (XL ​- Xc ) = 0 Minimum impedance! → Circuit behaves like a short circuit → AC current flows freely → Voltage across L and C may be large, but overall source sees low opposition Used in: - Band-pass filters - Tuned amplifiers - Signal selection 2️⃣ Parallel LC Circuit at Resonance Circuit: ¦----- L -----¦ AC Source ¦ ¦ back to source ¦----- C ---- ¦ What happens: - Each branch has current, but: - Inductor current lags - Capacitor current leads - At resonance: XL ​= Xc => equal and opposite - So, currents in L and C cancel each other out - → This means almost no net current is drawn from the source → From the source's point of view: circuit looks open → Hence, impedance is very high Used in: - Oscillators (tank circuits) - Band-stop filters - Frequency-selective circuits Summary Table Type; Resonance Behaviour; Impedance at Resonance; Current Drawn. Series LC; Reactances cancel; Minimum (Z → 0); Maximum. Parallel LC; Currents cancel; Maximum (Z → ∞); Minimum. |
| Low pass filter | Low pass filter - Active lpf : made using active components such as transistor or op-amp - Passive lpf : made using passive components (inductor , capacitor , resistor) 3 types of passive LPF RC Low pass filter RL low pass filter RLC low pass filter RC low pass filter; RL low pass filter. Vout= XCR2+Xc2 * Vin Here reactance : Xc = 12 π f C - At low freq Xc = ~∞ (acts as open ckt)(so we get vin at vout) - At high freq Xc = ~0 (acts as short ckt) (shunts the signal to gnd); Vout= RR2+XL2 * Vin Here reactance : XL = 2πfL - At low freq ( XL 0 ) (acts as short ckt)(so we get vin at vout) - At high freq ( XL ∞ ) (acts as open ckt) (so no current flows and no voltage occurs). Ideal Low pass filter response graph Actual low pass filter response graph At lower freq it provides 0 attenuation As the freq increases it gradually increases attenuation After cutoff freq (output will attenuate at the rate of -20dB Cut off freq formula for RC lpf fc= 12 π R C for RL lpf fc= R2 πL Output waveform will lag from the input phase by θ That is identified by below given equation ω=2 π f For RC low pass filter Cut off freq formula for RC lpf fc= 12 π R C θ= - tan-1(ωCR) = - tan-1(2 π f CR) - freq = 0 (DC) - tan-10=0° (op waveform Insync with input) - freq = fc (cutoff freq) - tan-11=-45° (op waveform lags by 45 degrees from input waveform) - freq = ∞ - tan-1∞= -90° (op waveform lags by 90 degrees from input waveform) Hence in short, the output waveform (for input of Vin and f freq is Vout and f freq with phase lag θ e.g. in input = 10v 2khz ac signal output will be Vout=6.22v , freq is same and phase shift of ~51° output waveform will have 6.22v of amplitude and freq will be same but will have phase lag of ~51°; For RL low pass filter Cut off freq formula for RC lpf fc= R2 πL θ= - tan-1(ωLR) = - tan-1(2 π f LR) - freq = 0 (DC) - tan-10=0° (op waveform Insync with input) - freq = fc (cutoff freq) - tan-11=-45° (op waveform lags by 45 degrees from input waveform) - freq = ∞ - tan-1∞= -90° (op waveform lags by 90 degrees from input waveform).. |
| High pass filter | High pass filter - Active hpf : made using active components such as transistor or op-amp - Passive hpf : made using passive components (inductor , capacitor , resistor) 3 types of passive hpf RC high pass filter RL high pass filter RLC high pass filter RC high pass filter; RL high pass filter. Vout= RR2+Xc2 * Vin Here reactance : Xc = 12 π f C - At low freq Xc = ~∞ (acts as open ckt)(so current doesn’t flow and no op appears) - At high freq Xc = ~0 (acts as short ckt) (current flows , op appears across resistor); Vout= XLR2+XL2 * Vin Here reactance : XL = 2πfL - At low freq ( XL 0 ) (acts as short ckt)(so we get vin at vout) - At high freq ( XL ∞ ) (acts as open ckt) (so no current flows and no voltage occurs). Ideal High pass filter response graph Actual low pass filter response graph At lower freq it provides increasing attenuation As the freq increases it gradually increases attenuation Before cutoff freq output will attenuate at the rate 20db/decade Cut off freq formula for RC hpf fc= 12 π R C for RL hpf fc= R2 πL Output waveform will lag from the input phase by θ That is identified by below given equation ω=2 π f For RC low pass filter Cut off freq formula for RC lpf fc= 12 π R C θ= tan-1(1ωCR) = tan-1(12πfCR) - freq = 0 (DC) tan-1∞=90° (op waveform leads by 90 degrees with input) - freq = fc (cutoff freq) tan-11=45° (op waveform leads by 45 degrees from input waveform) - freq = ∞ tan-1∞= 0° (op waveform in sync with input waveform); For RL low pass filter Cut off freq formula for RC lpf fc= R2 πL θ= tan-1(ωLR) = tan-1(2 π f LR) - freq = 0 (DC) tan-10=0° (op waveform Insync with input) - freq = fc (cutoff freq) tan-11=45° (op waveform leads by 45 degrees from input waveform) - freq = ∞ tan-1∞= 90° (op waveform leads by 90 degrees from input waveform).. |
| RC delay ckts *( One of capacitor application to delay voltage ) | It’s a circuit where a resistor and capacitor work together to delay a voltage change — typically when power is first applied or a switch is flipped. - When a step voltage (like 0 → 5 V) is applied at Vin, the capacitor charges slowly, not instantly. - Vout is taken across the capacitor, so you get a rising voltage with a delay. - The voltage across the capacitor follows an exponential charging curve. Vout= Vin*( 1- etRC ) Time; Vout (approx.). 0; 0 V. 1τ = RC; 63% of 5 V = 3.15 V. 2τ; 86% of 5 V. 3τ; 95% of 5 V. 5τ; ~100% of 5 V. This gradual increase creates a time delay before Vout reaches a certain logic threshold. 🛠️ Real-Life Example: Power-On Reset Delay - Some ICs or microcontrollers need time to stabilize after power is turned on. - You connect an RC delay at their reset pin. - Until Vout reaches a certain voltage, the IC stays in reset. - After the delay, Vout crosses threshold, and the IC starts running. |
| RC LPF as Integrator | It is basically LPF circuit (but in specific case it works are Integrator) - When Time period of input signal (T = 1f ) is less than τ=RC i.e. When T << τ or T << RC…LPF works as an Integrator (time period of waveform should be very lower than the RC(or tau)) Square wave triangular wave |
| RC differentiator ckt | It is basically hpf circuit (but in specific case it works are differentiator) When Time period of input signal (T = 1f ) is greater than τ=RC i.e. When T >> τ or T >> RC…HPF works as a differentiator (waveform’s time period should be very higher than the RC(or tau)) (tau=RC is very less means capacitor takes very less time to charge(charging time = 5*τ) and discharge Here in RC HPF – output is taken across resistor - When a square wave is applied - rising Edge at sudden input volt appears at the output ..however after sometime capacitor starts charging…making vout fall from Vin level to 0v level - const voltage output is blocked due to capacitor being fully charged… volt is 0 at output - falling edge at falling edge , capacitor encounters sudden drop in volt -so it tries to keep volt as it is and drains making vout appear high and then lessened as capacitor drained ) (As a result, it discharges rapidly, producing a negative spike at the output (opposite polarity)) |
| Band Pass filter | Series LC parallel LC LC circuit as band pass filter XL = 2 π f L Xc = 12πfC Band pass filter working concept - At low freq XL is lower ( effective impedance = capacitive impedance ( Xc ) - At high freq Xc is lower ( effective impedance = Inductive impedance ( XL ) - At certain freq (f resonance) - XL = Xc (so they cancel out each other – and effective impedance is very low) Resonance freq : fr= 12 π LC Series LC (as band-pass): - At resonant freq → XL = Xc → Total impedance = minimum (near zero) → Signal passes through = ✅ Band Pass behaviour Parallel LC (as band-pass): - At resonant freq → XL = Xc → Total admittance (1/Z) is max → Impedance is maximum = acts like an open circuit → Used in parallel with a resistor → maximum voltage across the LC → Resonance appears at output across LC tank So, it passes signal only if you take the output across LC or across a load connected in parallel. Type; Resonance Effect; Signal Behaviour. Series LC; Acts like a short (min impedance); Signal passes through. Parallel LC; Acts like an open (max impedance); Signal appears across LC. |
| Parasitic capacitance | Unwanted capacitance due to layout, materials, and device physics: - Gate Capacitance (Cg): Between MOSFET gate & channel (controls switching delay). - Interconnect Capacitance (Ci): Between adjacent wires (can cause crosstalk and delays). Unwanted capacitance – Cg + Ci + Cf(fanout gate capacitance) Effect: → Slows down signal transitions due to added RC delay (Capacitance charging/discharging causes delay) → Makes layout-sensitive → more critical in advanced nodes. |
| RC delay (gate delay) | - Every MOS gate behaves like an RC circuit: - R = Resistance of NMOS/PMOS channel - C = Load capacitance (parasitic + fanout gates) - Gate Delay ∝ RC → time to charge/discharge output node - Important for timing closure, affects clock frequency |
| Capacitors in ADCs, DACs & Sample-and-Hold Circuits | - Sample-and-hold (used in ADCs): → Capacitor samples input voltage and holds it steady while conversion happens. - Used to: - Maintain voltage level during conversion window. - Reduce signal fluctuation and improve ADC accuracy. |