Learning Path

Level 0: Oscillator Fundamentals (MUST-KNOW) Goal: Understand what an oscillator is and why it oscillates. Topics - What is an oscillator? - Oscillator vs amplifier - Need for oscillators in systems - Positive feedback concept - Barkhausen Criterion - Loop gain condition ¦Aβ¦ = 1 - Phase condition = 0° / 360° - Why oscillations start (noise) - Why oscillations stabilize (non-linearity) 👉 Learn via: Conceptual explanations + block diagrams

Level 1: RC Oscillators (Low-Frequency) Goal: Understand oscillators built using resistors & capacitors. Topics - RC phase-shift oscillator - Why 3 RC sections? - 180° phase shift from RC + 180° from amplifier - Wien bridge oscillator - Frequency of oscillation - Condition for oscillation - Automatic amplitude control (lamp / diode idea) - Frequency equations (important) - Advantages & limitations of RC oscillators 👉 Learn via: Circuit diagrams + phasor reasoning + simple math

Level 2: LC Oscillators (High-Frequency) Goal: Learn oscillators used at RF / MHz ranges. Topics - Why RC fails at high frequency - LC resonance concept - Tank circuit (energy exchange between L & C) - Hartley oscillator - Colpitts oscillator - Frequency of oscillation - Feedback mechanism in LC oscillators 👉 Learn via: Energy flow explanation + resonance concept

Level 3: Crystal Oscillators (Stability Focus) Goal: Understand why crystals are used for accurate clocks. Topics - Piezoelectric effect - Quartz crystal equivalent circuit - Series vs parallel resonance - Why crystal oscillators are ultra-stable - Typical frequencies (kHz–MHz) - Use in MCUs, CPUs, SoCs 👉 Learn via: Equivalent circuit + intuition (not fabrication)

General Level vs Expectation Summary Levelf; Needed for FE VLSI; Depth. Level 0; ✅ Mandatory; Strong. Level 1; ✅ Mandatory; Moderate. Level 2; ✅ Mandatory; Moderate. Level 3; 🟡 Useful; Conceptual. Completion Criteria You’ll be oscillator-ready when you can: - Explain Barkhausen clearly - Explain why RC/LC/crystal are chosen - Tell why ring oscillators are unstable but useful - Relate oscillator stability to digital timing

| Topic | Explanation |
| --- | --- |
| Basics | What is an Oscillator? : Definition (formal but simple) An oscillator is an electronic circuit that generates a periodic AC waveform (sine, square, etc.) from a DC power supply, without requiring any external input signal. 📌 Key idea: No input signal → yet you still get an output waveform. 🔌 Input–Output View (Very Important) Quantity; Oscillator. Input signal; ❌ None. Power supply; ✅ DC. Output; ✅ Periodic AC. So effectively: DC → (Oscillator) → AC 🧠 Why This Is Special Normally: - To get AC output → you need AC input But an oscillator: - Uses feedback - Uses energy storage - Uses device non-idealities to create AC on its own. 🧩 Simple Block Diagram (Generic Oscillator) Blocks: - Amplifier (Gain A) - Feedback network (β) → decides frequency - DC supply 🔁 Oscillator vs Amplifier (Crystal Clear) Feature; Amplifier; Oscillator. Input signal; Required; ❌ Not required. Output; Amplified input; Self-generated periodic signal. Feedback; Negative; Positive. Purpose; Increase signal strength; Generate signal. 🧪 Everyday Examples Device; Oscillator Inside?. Wall clock; ✅. Phone / laptop; ✅. MCU / FPGA; ✅. Radio; ✅. ⚠️ No oscillator = no clock = no digital system 🧠 What Waveforms Can Oscillators Generate? Oscillator Type; Output. Sinusoidal; Sine wave. Relaxation; Square / triangle. Examples: - RC, LC, crystal → sine (approximately) - Ring oscillator → square wave 🔑 One-Line Memory (Lock This) An oscillator is a circuit that converts DC power into a periodic AC signal using positive feedback, without any external input. 🔍 Tiny Reality Check (Very Important) You might think: “If there is no input, how does it start?” Answer (we’ll expand later): ➡️ Noise + positive feedback our microcontrollers use clock generated from crystal oscillator right - what kind of waveform is that inside the microcontroller, the clock is a DIGITAL SQUARE WAVE. 🧠 The Full Truth (3 layers) 1️⃣ The crystal itself A quartz crystal is a mechanical resonator. - It vibrates sinusoidally - Electrical behavior ≈ sine wave 📌 So at the crystal pins, the signal is: Small-amplitude, near-sine wave 2️⃣ The crystal oscillator circuit (inside MCU) Inside the MCU: - Crystal + inverter + capacitors form an oscillator - Output here is still: - Low amplitude - Analog - Sine-like / distorted sine 3️⃣ The clock distributed inside the MCU (IMPORTANT) The MCU cannot use a sine wave for digital logic. So internally it has: - Buffer - Schmitt trigger - Clock conditioning circuit This converts the analog oscillation into: ✅ Clean rail-to-rail square wave 📌 This square wave: - Drives flip-flops - Drives counters - Drives CPU pipeline - Drives timers 🧠 Summary Table (Remember This) Location; Waveform. Crystal vibration; Mechanical sine. Crystal pins; Low-amplitude sine-like. After buffer / Schmitt trigger; Square wave. Digital logic clock; Square wave. 🔑 One-Line Interview Answer Although a crystal oscillator produces a sinusoidal analog signal, microcontrollers internally convert it into a clean square-wave clock for digital logic. 🔍 Bonus Insight (VLSI relevance) Why not directly use sine wave? - Flip-flops need sharp edges - Timing depends on threshold crossing - Slow slopes cause: - Jitter - Metastability - Power loss So digital systems always use square clocks.. |
| What is an Oscillator | Link : https://www.youtube.com/watch?v=aJAZHPqEUKU - Oscillators in electrical means are voltages moving back and forth (like sine wave of 1v amplitude (moving from 1 to -1 and -1 to 1) - Oscillator can generate periodic signal (ex. - sine wave , triangular wave , square wave , etc) |
| Oscillator vs. Amplifier | Link : https://www.youtube.com/watch?v=E7C0g_XS_nU&list=WL&index=3 🔹 Amplifier’s goal Faithfully amplify an external input without changing its shape So we want: - Stable gain - Low distortion - Controlled output 👉 Negative feedback: - Opposes changes - Reduces sensitivity to variations - Prevents runaway - Improves linearity That’s why amplifiers use negative feedback. 🔹 Oscillator’s goal Generate a signal even when NO input exists So we want: - Signal to grow from noise - Continuous repetition - No decay 👉 Positive feedback: - Reinforces the signal - Adds output back in-phase - Allows noise to grow - Sustains oscillation That’s why oscillators use positive feedback.. |
| How oscillator works | Link : https://www.youtube.com/watch?v=LTsTl3db7Ko&list=WL&index=4 - What is an oscillator - An oscillator is something that creates oscillations which involves periodic fluctuations between two states or values - A clock pendulum is basically a mechanical oscillator - In electronics , we discuss the voltages that are fluctuating from a certain value to value - So an oscillator is a ckt that generates repetitive waveforms without any input - Oscillator can produces signal as shown below - Electronic oscillators are used in wide ranges of applications – such as watches , computer , radios,etc. - Few other examples - Clock generation (most important use case) - Where: Digital circuits need a timing reference to know when to execute instructions. - Microcontrollers (Arduino, STM32, AVR) - CPUs, GPUs, FPGAs, ASICs - Example : 16 MHz crystal on Arduino , 100 MHz system clock in a CPU - Communication systems (RF & wireless) - Where : To generate carrier frequencies and local oscillator (LO) signals. - Mobile phones - Wi-Fi, Bluetooth - Radios, GPS - Example : Phone uses oscillators at GHz range , FM radio uses oscillator to tune stations - Oscillator decides which frequency you transmit/receive - Timekeeping (watches & RTCs) - Where : To keep accurate time. - Digital watches - RTC chips (DS1307, DS3231) - Phones, laptops - Example : 32.768 kHz crystal → divided down to 1 Hz - Audio & sound generation - Where : To generate audible frequencies (20 Hz – 20 kHz). - Buzzers - Musical keyboards - Alarm systems - Example : Beep sound = square wave oscillator , Synth = multiple oscillators (sine, sawtooth) - Sound = vibration = oscillator output - Power electronics & SMPS - Where : To create PWM switching signals. - Buck / Boost converters - Mobile chargers - Laptop adapters - Example : 100 kHz–1 MHz oscillator drives MOSFET , Duty cycle controls output voltage - No oscillator → no switching → no regulation - Display & video systems - Where : To generate pixel clock and synchronization timing. - LCD / LED displays - HDMI / VGA - Camera sensors - Example : Pixel clock decides refresh rate , Wrong clock → flicker or distorted image - Oscillator controls visual stability - Measurement & test equipment - Where : To generate known reference signals. - Function generators - Oscilloscopes - Frequency counters - Example : Sine wave generator = oscillator , Time base in CRO = precision oscillator - Security & digital systems (VLSI angle) - Where : To generate pixel clock and synchronization timing. - PLLs - Clock tree synthesis - Random number generators - Why oscillator is needed: - PLL starts with oscillator - Ring oscillators used for on-chip variation detection - Oscillator = foundation of digital silicon - An oscillator receives DC power and produces AC signal as output (it can generate freq from Hz to GHz) - Basic oscillator ckt - Using inductor and capacitor (LC oscillator) - Take a capacitor – charge it and then connect an inductor - You will see continuous flow of energy between inductor and capacitor - This creates oscillations - Oscillator Using 555 timer IC How does an oscillator work - Practically oscillator is an amplifier with positive feedback - Working principle - When input is applied to amplifier it amplifies the signal (so for input = Vin output is A*Vin (where A is gain)) - Here the β = feedback fraction and the feedback section is referred as “freq selective ckt” or “resonance ckt” (β controls the fraction – that how much part of the output is to be fed back to input) - If the input and output is in phase with each other (0-degree phase shift from input) - If we add this inphase output to input we would be giving positive feedback to the input – and the input signal becomes Vin + Vf ( so this is basically amplifier with positive feedback) - Now if we remove/disconnect input signal Vin - only Vf will be present at the input (this will act as its input) - After removing input ,weather we will get sustained oscillation or not depends on the “loop gain” = A*β - If “loop gain” = A*β < 1 - Electrical oscillation amplitude will go on decreasing over a period of time and eventually die (this is called damped oscillation ) - Example : - Let’s say A*β=0.6 and the input volt is 3v - So, after 1st cycle op = 1.8v - So, after 2nd cycle op = 1.08v - So, after 3rd cycle op = 0.648v and so on … - If “loop gain” = A*β > 1 - Electrical oscillation amplitude will go on increasing over a period of time and after sometime it will reach the rails (Vcc to -Vcc) (damped oscillation) - Example : - Let’s say A*β=1.2 and the input volt is 3v - So, after 1st cycle op = 3.6v - So, after 2nd cycle op = 4.32v - So, after 3rd cycle op = 5.18v and so on … - If “loop gain” = A*β = 1 - Here the output will be same as input – hence we will get sustained oscillation { undamped oscillation} Barkhausen criterion - For sustained oscillations Barkhausen criterion must be followed - ¦ Aβ ¦=1 (loop gain = 1) - ∠Aβ=0° or 360° (no phase shift between input and output) [ note : here we have shown that initially an input voltage is given – but that is just for understanding – in actual no input is given – but the op-Amp (as amplifier) picks up the thermal noise as input (which is always present) - In reality – this is what happens - Thermal noise gets picked up as input and is amplified (noise has many signals of different frequencies – all of them gets amplified and fed to the feedback ckt) - Which is then fed to feedback ckt – which is also a freq selective ckt – so out of many only few frequencies get selected - And out of all these frequencies only 1 has 0 phase shift (so only one left) - Here initially the loop gain (Aβ) is kept > 1 to increase the volt level of noise signals – until it reaches the volt level we want – and the loop gain (Aβ) is again changed to 1 - Output Vout=A*( Vin+ βVout ) An oscillator oscillates at the frequency where BOTH conditions are met: - Phase = 0° - Loop gain ≥ 1 👉 Important insight: - Many frequencies start (from noise) - Feedback network kills all except one - That one frequency satisfies Barkhausen criteria. Why only one frequency survives from noise? - Thermal noise contains many frequencies - All random frequencies are initially amplified - The feedback network is frequency-selective - It provides different phase shifts and gains at different frequencies - Only one frequency satisfies BOTH conditions - Net phase shift = 0° / 360° - Loop gain ≥ 1 (initially) - All other frequencies die out - If phase ≠ 0°, feedback becomes negative → attenuation - If loop gain < 1 → amplitude decays - The surviving frequency grows - Until non-linearity reduces loop gain to exactly 1 - Result → stable oscillation at one frequency. Thermal noise (also called Johnson–Nyquist noise) is the random electrical noise generated due to heat in any conductor or electronic component. In simple words Because of temperature, electrons inside a material are always moving randomly. This random motion causes tiny, unpredictable voltage and current fluctuations — even when no signal is applied. 👉 If something has temperature above 0 K, it has thermal noise. Key characteristics - ✅ Always present (can’t be eliminated) - ✅ Exists even in resistors, wires, ICs - ❌ Independent of applied voltage or current - 📈 Increases with temperature - 📈 Increases with bandwidth - 🎲 Completely random (white noise=White noise is a random signal that has equal power at all frequencies within a given bandwidth.) Where it comes from - Random collisions of electrons with atoms - Purely a physical phenomenon, not a circuit fault. Oscillator types |
| How oscillator works | Link : https://www.youtube.com/watch?v=skDVcnVwK3A&list=WL&index=9 Two common oscillators - LC oscillator ckt - RC oscillator ckt - LC oscillators - Energy transfer takes place between capacitor and inductor and we get sinusoidal waves - Freq of oscillations fr= 12π LC - Common LC oscillators - Heartley oscillator (feedback via two inductors (or a tapped inductor) + one capacitor) - Colpitts oscillator (feedback via two capacitors + one inductor) - RC oscillators - Freq of oscillations fr= 12π RC ( specific oscillators have their own variations of this formula – this one is general) - Common RC oscillators - RC phase shift oscillator (uses RC network and a transistor / or an opAmp) - fr= 12π RC2N - Wein bridge oscillator (uses network and an opAmp) - fr= 12π RC - 555 timer (generates square wave) – - can be converted to sine wave using additional LC network as shown below - can be converted to triangular wave using additional RC network How LC oscillator work and How RC oscillator work 1️⃣ Why LC feels intuitive You already get this: - Inductor stores energy → magnetic field - Capacitor stores energy → electric field - Energy shuttles back and forth → oscillation 👉 RC has no inductor, so no magnetic energy storage → “then how oscillation?” Valid question. 2️⃣ Key mindset shift (VERY IMPORTANT) RC oscillators do NOT oscillate by energy exchange. They oscillate by phase shift + amplification. This is the core difference. 3️⃣ What RC actually does Step-by-step intuition - Capacitor charges and discharges - This causes time delay - Delay = phase shift - RC networks create frequency-dependent phase shift - At one particular frequency: - Total phase shift = 180° - Amplifier provides the other 180° - Net phase shift = 360° → positive feedback - Amplifier supplies energy - RC network does not store energy - Amplifier keeps re-injecting energy every cycle 👉 Oscillation is maintained because feedback stays in phase, not because energy swaps. 4️⃣ Compare LC vs RC (THIS WILL LOCK IT) Feature; LC Oscillator; RC Oscillator. Energy storage; L + C; Only C. Oscillation mechanism; Energy exchange; Phase shift + gain. Frequency set by; Resonance; RC time constant. Typical frequency; High (kHz–GHz); Low (Hz–hundreds of kHz). Loss compensation; Small; Larger (amp driven). 5️⃣ One powerful analogy (remember this) LC oscillator 👉 Pendulum (natural back-and-forth) RC oscillator 👉 Person pushing a swing at the right time - Swing alone would stop - Correct timing (phase) + repeated push = oscillation. |

| RC phase shift oscillator (RC oscillator) | RC Phase-Shift Oscillator — Overview 🔹 What it is : A sinusoidal oscillator that uses: - An inverting amplifier - A 3-stage RC phase-shift network (HPF type) The RC network provides 180° phase shift, and the amplifier provides the remaining 180°, satisfying Barkhausen criteria. 📏 Frequency Range Practical frequency range : ~10 Hz to ~100 kHz (typically range of audio frequencies) Why this range? - At low frequency → requires very large R or C (impractical) - At high frequency → op-amp phase lag & RC parasitic dominate Frequency formula (for N identical RC sections) f0=12πRC2N - R, C → set frequency only - Gain → does NOT affect frequency ✅ Use Cases (Where it is actually used) 1️⃣ Audio-frequency signal generation - Tone generators - Beep / alarm circuits - Educational audio oscillators 2️⃣ Function generators (basic) - Low-cost sine wave source - Lab demonstrations 3️⃣ Analog learning & teaching - Phase shift concept - Barkhausen criteria demonstration - Startup vs steady-state behavior 4️⃣ Low-frequency clock generation - When crystal or LC is not required - Simple timing references 5️⃣ Precursor to VLSI ring oscillators - Conceptually similar: - Delay → phase shift - Gain → sustain oscillation ❌ Cons / Limitations (Very important) ❌ 1. Requires high gain (≥29) - Heavy attenuation in RC network - Amplifier must provide large gain - Increases distortion risk ❌ 2. Poor amplitude control - No inherent amplitude stabilization - Amplitude limited by: - Op-amp saturation - Nonlinearity - Leads to distortion ❌ 3. Not suitable for high frequencies - Op-amp phase lag breaks phase condition - RC parasitics dominate ❌ 4. More distortion than Wien bridge - Wien bridge has built-in amplitude control - RC phase shift oscillators usually have higher THD(total harmonic distortion) ❌ 5. Startup sensitive - Needs: - Gain margin - Noise / initial imbalance - In simulation, often needs: - “Skip initial operating point” - Used in low freq sine waveform generation upto KHz - Amplifier provides 180-degree phase shift (opAmp or transistor is used ((common-emitter BJT or inverting op-amp)) ) and thus feedback ckt needs to be in such a way that it should provide 180-degree phase shift (as per Barkhausen criteria) - Now let’s dig the RC circuit - This is a classic “HPF using RC network” [ Xc=reactance= 12πfc ] - when low freq signal is passed – capacitor provides high reactance , acting as open ckt – no current flows – no output appears at output - when high freq signal is passed – capacitor provides low reactance , acting as short ckt – current flows – output appears across R - The phase shift provided by this ckt ∠θ = tan-1( XcR ) - For very small values of Xc (high freq), phase shift will be 0 degree (Xc=0) - For high values of Xc(low freq) , phase shift will be 90 degrees (Xc=∞) - So, this ckt provides phase shift from 0° to 90° - So theoretically to get 180° phase shift we need to cascade two RC network - But what HPF offers - At low freq ( Xc ↑ ) means provides 90-degree phase shift (but magnitude = 0) (as Xc is high - capacitor acts as open ckt and signal doesn’t go ahead – making nothing appear at output) (so 90-degree phase shift also comes with no signal for further stage) - At High freq ( Xc ↓ ) means provides 0-degree phase shift (as Xc is low - capacitor acts as short ckt and signal goes ahead and appears across resistor–) - thus 180° phase shift using two 90° phase shift RC stages are not practical [ hence 3 RC stages are used – each providing 60° phase shift (here each won’t provide exact 60° because of the loading effect of prev stages – but the overall would become 180° ) - This way by using more than 2 stages one can achieve 180° - Example : 3 stages of 60° , 4 stages of 45° RC phase shift Oscillator using OpAmp - The attenuation provided by RC circuit is β=- 129 (represent 180-degree phase shifted value (hence ‘-‘ sign)) - So, the gain provided by Amplifier must be A = 29 - Now , this oscillator ckt only provides 0/360-degree phase shift and unity gain at one freq - f= 12πRC 2N {N=number of RC stages}{example – in above image it is 3} - in RC phase shift oscillator – R is kept fix and C is made changeable (that is how u can set the freq of oscillation) |
| --- | --- |
| Wein bridge oscillator (RC oscillator) | Link : https://www.youtube.com/watch?v=gbUXbaxvX94&t=576s Wien Bridge Oscillator — Frequency Range 🔹 Practical frequency range : ≈ 10 Hz to 1 MHz 🔹 Most commonly used / best-performance range : 100 Hz to 100 kHz This is where it is: - Most stable - Lowest distortion - Easiest to control amplitude 🧮 Frequency formula For equal R and C in the Wien network: f0=12πRC So: - Low frequency → large R or large C - High frequency → small R and small C - opAmp is used in non-inverting config - Here positive feedback can be seen (highlighted part is the RC feedback network) - Series RC network acts like HPF and parallel RC network acts like LPF - Series RC network as HPF - At low freq (XC↑) capacitor acts as open ckt (blocks the current from flowing – no output appears - At high freq (XC↓) capacitor acts as short ckt allows passing of high freq signal - In this way it acts as HPF - Parallel RC network as LPF - At low freq (XC↑) capacitor acts as open ckt (thus, entire drop appears across R2 ) – allowing Vin to appear at output - At high freq (XC↓) capacitor acts as short ckt allows passing of high freq signal to gnd by providing low resistance path (basically shunts it) - In this way it acts as LPF - So, this RC network doesn’t allow high as well as low frequencies but allows middle range of frequencies (out of which only at 1 particular freq output will be max-which is the resonant freq) (at rest of the frequencies - output will be minimum) (bell type curve – output max at one point only) - At resonant freq - phase shift will be 0 - Ratio of VoutVin= 13 = β - When R1 = R2 and C1 = C2 then fr= 12πRC - When R1 ≠ R2 or C1 ≠C2 then fr= 12πR1R2C1C2 - To satisfy Barkhausen criteria ¦Aβ¦ = 1 ( as β=13 , A should be 3) - Here A = amplifier gain = 1+ RfR1 =3 (which when solved states Rf=2R1) Amplitude stabilization Link : https://www.youtube.com/watch?v=-AEkdXj2H7A&list=WL https://www.youtube.com/watch?v=AWpYtYj-pQw&list=PLXb3r5ny8_1W6sIWl6CWFOy-F31sAt9j_&index=2 what we want - - When the Amplitude is higher , lower the gain - When amplitude is lower , higher the gain Method 1 : using tungsten lamp - Here when oscillation amplitude increases , temperature of lamp increases resistance of lamp increases close loop gain decreases (as for Wein bridge oscillator – gain = ( 1 + RfR1 ) ) Method 2 : using diode clamp Reimagined structure : - Here two diodes are parallel and in opposite direction with each other (they are placed like that because we need at least one ON in positive and negative cycle) - R4 is in series with this diode clamp structure - For ideal amplifier – both the input is at same potential (so same appears at the junction as shown in above restructured image) - Now, - Let’s say signal is of 1mV output will be of 3mV (the difference between this two is insufficient to turn on the diode)(for silicon diode we need at least 0.7v) : so, with small signals – diodes are off - Small signal diode off Amplifier gain = ( 1+ RfR1 ) - Let’s say signal is of 0.5v output will be of 1.5V (the difference between this two is sufficient to turn on the diode) : so, with large signals – any one diode is ON (because for positive part of the cycle any one is conducting and for negative half of the cycle any other one is conducting) - large signal any one diode is ON Amplifier gain = ( 1+ Rf ¦¦ ( R3+RD) R1 ) [RD=diode resistance) - Rf ¦¦ ( R3+RD) will always be < Rf (if u put anything in parallel with anything the effective resistance is lesser than both the resistor) - Basically, what we are doing here - When the signal is small – we provide higher gain - When the signal is big – we provide lower gain |
| Advantages and limitation of RC oscillator | What are RC oscillators (1-line recap) RC oscillators use resistors (R) and capacitors (C) to generate low-frequency sinusoidal oscillations. Common types: - RC phase-shift oscillator - Wien bridge oscillator Advantages - Simple and low cost - Ideal for audio and low freq range (typically 10Hz to 100KHz) {LC oscillator struggles at very low freq} - Stable sinusoidal output (especially Wien bridge) [Wien bridge oscillator gives: low distortion , clean sine wave with amplitude control] - Easy frequency tuning (using R and C) Limitation - Limited freq range - Lower freq accuracy (Depends on RC tolerances , Temperature & aging affect) - Lower output power ( RC oscillators produce small signal levels - Need additional amplifier for driving loads) - Not suitable for precision timing (Not used where: Clock accuracy is critical , Long-term stability is needed (That’s why microcontrollers don’t use RC oscillators for clocks (they use crystals)) Typical applications - Audio signal generators - Function generators - Test & measurement equipment - Low-frequency waveform sources - Educational & lab circuits In short : RC oscillators are simple, low-cost, and ideal for low-frequency sine wave generation, but they lack frequency precision and high-frequency capability. |
| Why do oscillations stabilize due to non-linearity? | What happens step-by-step: - At startup - Noise provides a tiny signal - Loop gain ¦Aβ¦ > 1 - Oscillation amplitude grows exponentially - As amplitude grows - Active device (BJT / MOSFET / op-amp) leaves linear region - Enters non-linear region - BJT → saturation / cutoff - Op-amp → rail limitation - Effective gain reduces - Small-signal gain ≠ large-signal gain - Gain automatically drops as amplitude increases - At steady state - Gain settles such that: ∣Aβ∣=1 - Oscillations neither grow nor decay 🔑 Key idea Non-linearity acts as an automatic gain control that limits amplitude and stabilizes oscillations. That’s the heart of oscillator theory. |
| Why RC oscillators fail at high frequency | Why RC oscillators fail at high frequency RC oscillators fail at high frequency because RC networks introduce excessive phase error, attenuation, and parasitic dominance, making it impossible to satisfy the Barkhausen condition reliably. 1️⃣ RC networks rely on resistance → energy loss In an RC circuit: - Capacitor stores energy - Resistor dissipates energy as heat At every cycle, energy is lost. At low frequency: - Loss per cycle is manageable - Amplifier can compensate At high frequency: - Energy loss per second becomes huge - Amplifier gain requirement increases dramatically 🔑 Result: Loop gain ¦Aβ¦ drops below 1 → oscillations die. 2️⃣ Phase accuracy collapses at high frequency (this is critical) RC oscillators depend on precise phase shift. Example: - RC phase-shift oscillator needs: - 180° from RC - 180° from amplifier But at high frequency: - RC phase shift becomes frequency-sensitive - Small changes in R or C → large phase error ⚠️ Even 5–10° phase error breaks oscillation. 🔑 Result: Phase condition (0°/360°) cannot be maintained 3️⃣ Parasitic components dominate at high frequency At high frequency: - Resistors have parasitic inductance - Capacitors have: - ESR (series resistance) - ESL (series inductance) - PCB traces behave like inductors So your “RC” network becomes: R + L + C (uncontrolled) 🔑 Result: - Unpredictable resonance - Unstable frequency - Distorted waveforms 4️⃣ Required amplifier gain becomes impractical For RC phase-shift oscillator: - Minimum gain ≈ 29 At higher frequency: - RC attenuation increases - Required gain may exceed: - Op-amp bandwidth - Transistor fT 🔑 Result: Active device cannot provide required gain at that frequency 5️⃣ Op-amp & transistor bandwidth limits Even if RC network “theoretically” works: - Op-amps have finite gain-bandwidth product - Phase lag from amplifier itself increases at high f So: - Total loop phase ≠ 360° - Barkhausen fails 6️⃣ Frequency drift becomes severe At high frequency: - Temperature changes - Component tolerances - Supply variation All cause: - Large frequency shifts - Poor repeatability |
| Series resonance vs parallel resonance | Link : https://www.youtube.com/shorts/30KLkeL0_DY 1. Series Resonance (Left Side) Circuit - Resistor R, Inductor L, and Capacitor C are connected in series with an AC source. - Same current flows through all components. Impedance Expression Z=R+jωL1ωC - ωL→ inductive reactance - 1ωC→ capacitive reactance - Net reactance depends on frequency Resonance Condition At resonance: ωL=1ωC So the reactive part becomes zero: Z=R 👉 Circuit behaves as purely resistive Impedance vs Frequency Plot - At low frequency → capacitive → impedance high - At high frequency → inductive → impedance high - At resonance (ωr) → impedance is minimum (= R) Current vs Frequency Plot - Since impedance is minimum at resonance: - Current is maximum at ωr - This is why series RLC is called a current-resonant circuit Resonant Frequency fr=12πLC 2. Parallel Resonance (Right Side) Circuit - Resistor R, Inductor L, and Capacitor C are connected in parallel across the AC source. - Same voltage appears across all branches. Admittance Expression Y=1R+jωC1ωL - Admittance is the inverse of impedance - Reactive admittance depends on frequency Resonance Condition At resonance: ωC=1ωL Reactive admittance becomes zero: Y=1R Thus, Z=R Impedance vs Frequency Plot - At resonance: - Inductive and capacitive branch currents cancel - Source sees maximum impedance - Hence parallel RLC is called an impedance-resonant circuit Current vs Frequency Plot - Since impedance is maximum at resonance: - Source current is minimum at ωr - Large currents may circulate internally between L and C Resonant Frequency fr=12πLC (Same expression as series RLC) 3. Key Observations from the Diagram Aspect; Series RLC; Parallel RLC. Resonance effect; Reactances cancel in series; Reactive currents cancel. Impedance at resonance; Minimum (= R); Maximum (= R). Source current; Maximum; Minimum. Dominant nature; Current resonance; Impedance resonance. Final One-Line Memory Aid - Series resonance → minimum Z → maximum I - Parallel resonance → maximum Z → minimum I Note : the sharpness of the impedance curve depends on the Q-factor |
| LC oscillator | Link : https://www.youtube.com/watch?v=2_y_3_3V-so https://www.youtube.com/watch?v=nh4q7mIhLrY&t=274s Oscillation freq range : ~20 kHz to ~30 MHz Why this range? The Hartley oscillator is an LC oscillator, so its frequency is set by an inductor–inductor–capacitor (L₁, L₂, C) tank: f0=12π(L1+L2) C - Lower limit (~20 kHz) - Needs large inductors → bulky, lossy, unstable - RC oscillators are better here - Upper limit (~30 MHz) - Parasitics (wiring capacitance, transistor internal capacitance) dominate - Coil self-resonance + layout issues kill stability Practical usage bands Frequency band; Suitability. Audio (<20 kHz); ❌ Poor. Low RF (100 kHz – 5 MHz); ✅ Excellent. HF RF (5 – 30 MHz); ✅ Very common. VHF (>30 MHz); ⚠️ Not preferred. Where Hartley is actually used - RF signal generators - Amateur radio (HF band) - Local oscillators in older receivers - Tunable RF oscillators (easy coil tapping) - In the 1st ckt – a charged capacitor is providing the current to inductor (in graph 1st half of upper signal) ( current vs time graph) - In the 2nd ckt – now inductor becomes more charged than capacitor (hence to maintain the dropping current – it starts to collapse its voltage in order to keep the current same (u see it in the 2nd half of the upper signal ) - Now the charged capacitor has reverse polarity ( as inductor has charged it in other way) – so the current the capacitor gonna provide to inductor will be the negative 1st half of the graph - And then inductor again provides current after capacitor charge is drained will be the 2nd half of the negative cycle - This is how LC tank works (by exchanging energy continuously ) Energy Storage in LC Circuit An LC circuit has no resistance ideally, so it does not dissipate energy. Instead, energy oscillates between: - Capacitor (C) → stores energy in the electric field EC=12CV2 - Inductor (L) → stores energy in the magnetic field EL=12LI2 Energy Exchange Mechanism - Initially, the capacitor is charged → energy stored in electric field - Capacitor discharges through inductor → current builds up - Inductor stores energy in magnetic field - Magnetic field collapses → charges capacitor with opposite polarity - Cycle repeats 👉 Energy keeps circulating between L and C Key Characteristics of a Tank Circuit - Stores oscillating energy - Has a natural resonant frequency fr=12πLC - At resonance, energy exchange is maximum - Losses are minimal (ideally zero) How to pick L & C [General Guidelines to Choose L and C for heartley and colpitt oscillator] Note : SRF (self-resonant frequency) – SRF (Self-Resonant Frequency) is the frequency at which an inductor stops behaving like an inductor and starts behaving like a capacitor and a capacitor behaves like an inductor. 1. Start from the required oscillation frequency For any LC oscillator, frequency is set by resonance: - Hartley oscillator f0=12π(L1+L2) C - Colpitts oscillator f0=12πL⋅Ceq where Ceq=C1C2C1+C2 👉 Frequency requirement is fixed first; L and C are chosen to satisfy it. 2. Prefer larger C and smaller L This is the most important practical rule. Why? - Capacitors have lower losses than inductors - Large inductors → higher winding resistance → lower Q - Smaller inductors → higher SRF and better RF behaviour Design preference: - Use moderate to large capacitance - Use minimum inductance needed for the frequency 3. Ensure high Q of the LC tank Q=2π×Energy storedEnergy lost per cycle - High Q → stable frequency, low phase noise - Low Q → oscillation decay or distortion Component choices: - Air-core inductors preferred - Low DC resistance (DCR) inductors - C0G / NP0 ceramic capacitors only 4. Check Self-Resonant Frequency (SRF) Both L and C have SRF due to parasitics. Rule of thumb: fSRF≥5–10×f0 - Inductor SRF is usually the limiting factor - Operating near SRF causes phase error, loss of Q, and instability 5. Hartley-specific considerations - Total inductance = L1+L2 - Feedback ratio depends on inductive tap Feedback fraction∝L1L2 Guidelines: - Use single tapped inductor if possible - Avoid very unequal L₁ and L₂ (causes poor feedback) - Keep total inductance low for high-frequency operation 6. Colpitts-specific considerations - Feedback is set by capacitive divider Feedback fraction=C1C2 Guidelines: - Typical starting point: C2=5–10×C1 - Use identical capacitor types for C₁ and C₂ - Avoid very small capacitor values (parasitics dominate) 7. Avoid loading the LC tank - Do not connect low-resistance loads directly across the tank - Use buffer stages or take output from amplifier node - Loading reduces Q and shifts frequency 8. Bias network must not steal AC energy - Use RF choke (inductor) instead of resistor where possible - Bias components should appear: - Low impedance to DC - High impedance to AC 9. Typical component ranges (practical) Frequency range; Capacitor; Inductor. 100 kHz – 1 MHz; 100 pF – 10 nF; 1 µH – 100 µH. 1 MHz – 10 MHz; 10 pF – 1 nF; 100 nH – 10 µH. 10 MHz – 50 MHz; 5 pF – 100 pF; 50 nH – 1 µH. 10. Golden summary (exam + design ready) “In Hartley and Colpitts oscillators, L and C are chosen to set the desired resonant frequency while maximizing Q factor. This is achieved by using low-loss capacitors, minimizing inductance value and resistance, ensuring SRF is much higher than the operating frequency, maintaining proper feedback ratio, and avoiding loading of the LC tank.” 🔑 One-line engineering intuition Oscillators don’t want perfect formulas — they want low loss, high Q, and clean energy exchange.. |
| Heartley Oscillator | Link : https://www.youtube.com/watch?v=7ICMo0NeL_4&t=1s https://www.youtube.com/watch?v=3B_sBX_11Zw Note : on YouTube many videos are available but they don’t explain properly – here is how I understood it (using ChatGPT) 1. What is Ground in general? Ground is a reference node against which voltages are measured. It is not automatically a place where current disappears. 2. DC Ground Definition : DC ground is a node that is fixed at 0 V for DC (steady-state). How it is created - Direct electrical connection to: - Battery negative - Power supply return - Chassis / earth (sometimes) Behaviour - DC current can flow into and out of this node - Used for: - Biasing - Setting operating points - Providing return path for DC currents Example In a CE amplifier: - Emitter resistor is connected to DC ground - Sets emitter current and bias 3. AC Ground Definition : AC ground is a node that is held at 0 V for AC signals, but not necessarily 0 V for DC. This is the subtle part. How AC ground is created Usually by a large capacitor connected to ground. For AC: XC=1ωC→very small So: - AC sees near-short to ground - Voltage at that node ≈ 0 (for AC) For DC: - Capacitor is open circuit - DC voltage at that node is unaffected Behaviour - AC voltage ≈ 0 - AC current can pass - DC voltage can be non-zero - DC current cannot pass through capacitor 4. Why AC Ground is needed In amplifiers and oscillators, we want: - Stable DC bias - Free AC signal movement AC ground lets us: - Lock DC operating point - Remove AC signal from that node 5. Example: CE Amplifier Emitter - Emitter resistor sets DC bias - Bypass capacitor across emitter resistor: For DC: - Capacitor open - Resistor controls bias For AC: - Capacitor short - Emitter is AC ground - Gain increases 6. AC Ground in Hartley Oscillator - The inductor tap is connected to AC ground - This: - Fixes phase reference - Does NOT absorb AC energy - Does NOT break LC tank The oscillating current still flows in a closed loop via L and C. 7. Key Differences (Table) Feature; DC Ground; AC Ground. Voltage for DC; 0 V; May not be 0 V. Voltage for AC; May vary; ≈ 0 V. DC current; Allowed; Blocked. AC current; Allowed; Allowed. Created by; Wire; Capacitor. 8. Common Misconception (Important) ❌ Ground = current sink ✅ Ground = voltage reference Current always flows in loops, not into ground. 9. One-line notes statement ✍️ DC ground is a reference point fixed at 0 V for steady-state operation, while AC ground is a node held at zero AC voltage using capacitors, allowing AC signals to bypass without disturbing DC bias. Ultra-short intuition (lock this) - DC ground → where bias is set - AC ground → where signal disappears (voltage-wise). Hartley Oscillator 1. Definition A Hartley oscillator is an LC oscillator in which the feedback required for sustained oscillations is obtained from a tapped inductor (or two inductors in series) forming an inductive voltage divider, along with a capacitor forming the LC tank. Typical frequency range: ➡️ ~30 kHz to ~30 MHz 2. Circuit Components - Amplifier: Common-Emitter (CE) BJT amplifier - Tank circuit: - Inductors L1and L2(parts of one tapped inductor) - Capacitor Cconnected across L1L2 - Tap point: Connected to AC ground (via bypass capacitor) 3. Role of Each Element (a) LC Tank Circuit - Stores energy alternately in: - Inductor → magnetic field - Capacitor → electric field - Determines oscillation frequency: fr=12π(L1+L2)C (b) Tapped Inductor (L₁ and L₂) - L1and L2together form one resonant inductor - The tap provides: - A fraction of the tank voltage for feedback - Opposite polarity at the tap (used for phase inversion) - Acts as an inductive voltage divider: β=VL1VL2=L1L2 (c) CE Amplifier - Provides required gain - Introduces 180° phase shift - Supplies energy to compensate for tank losses 4. Working Principle (Time-Domain View) - When power is applied, noise or small transients appear in the circuit. - The CE amplifier amplifies these signals, producing an inverted (180°) output at the collector. - The collector drives the LC tank. - Current first builds in the inductor (L2+L1), storing magnetic energy. - The changing current develops voltage across the capacitor, storing electric energy. - Energy oscillates between L and C at the resonant frequency. - A fraction of the tank voltage appears across L2and is fed back to the base. - This feedback reinforces the original signal, causing oscillations to grow. - Oscillations stabilize once energy supplied equals losses. 5. Phase Condition (Barkhausen) For sustained oscillations, total phase shift around the loop must be 0° or 360°. - CE amplifier → 180° phase shift - Tapped inductor (polarity at tap) → 180° phase shift Total phase shift=180∘+180∘=360∘ ✔ Phase condition satisfied 6. Gain Condition (Barkhausen) Barkhausen gain criterion: Aβ=1 For Hartley oscillator: β=L1L2⇒A=L2L1 - At startup: Aβ>1→ oscillations grow - At steady state: Aβ=1→ constant amplitude ✔ Gain condition satisfied 7. Role of AC Ground - The inductor tap is connected to AC ground - AC ground: - Fixes AC voltage reference - Does not absorb AC current - Oscillating current flows in a closed loop through L and C 8. Why Oscillations are Sustained - LC tank alone produces damped oscillations due to losses. - The amplifier continuously supplies energy equal to losses. - Thus, steady sinusoidal oscillations are maintained. 9. Key Characteristics - Frequency depends on L1+L2and C - Simple construction - Uses inductive feedback - Less stable than crystal oscillators AC ground affects: - Voltage → fixed at 0 (AC) AC ground does NOT affect: - Current continuity - Energy circulation So: - Voltage at tap ≈ 0 - But current flows through that node Analogy (lock this in) Imagine a round track: - AC ground = a milestone labelled “0” - Current = runners on the track Runners pass the milestone; they don’t disappear there. Why DC bias is preserved Because the capacitor used for AC ground: - Is open circuit for DC - So DC current cannot flow - DC bias remains unchanged Thus: - DC bias → stable - AC signal → free to circulate. |
| Colpitts oscillator | Link : https://www.youtube.com/watch?v=1fgw-ONlAcc&t=400s LEARN HEARTLY Oscillator first if u r visiting this Typical frequency range: ➡️ ~100 kHz to ~100 MHz (practically most stable from 1 MHz to 30 MHz) Colpitts oscillator with CE amplifier (using transistor) Colpitts oscillator with Inverting amplifier (using Op-Amp) Note : here , transistor Amplifier (CE amplifier) is used (we can use op-Amp as well) f= 12πLCeq (freq of oscillation / resonant freq) Ceq= C1C2C1 +C2 A = (required gain of Amplifier) = C2C1 β = (Gain of feedback network) = C1C2 |
| Why do RC oscillators struggle at MHz frequencies but LC oscillators don’t? | At higher freq phase shift is almost 0… RC phase shift needs 180° from feedback. Assessment: ✔️ Directionally correct ⚠️ Missing the real killer reasons Complete engineer answer (lock this in): RC oscillators struggle at MHz because: - Parasitic capacitances dominate (transistor, wiring, PCB) - RC phase shift becomes inaccurate - Heavy attenuation → impractically high gain needed - RC networks dissipate energy, they don’t store it LC oscillators: - Store energy (magnetic + electric) - Work with high frequency physics - Naturally resonate at MHz 🔑 One-line memory: RC fights high frequency; LC thrives on it. 🔑 Why LC oscillators work well at high frequency 1️⃣ LC stores energy, RC dissipates it - LC: - Inductor → magnetic energy - Capacitor → electric energy - Energy is recycled - RC: - Resistor burns energy as heat - Loss increases with frequency At high frequency: - RC losses explode - LC losses remain small 2️⃣ LC resonance is a natural high-frequency phenomenon Resonant frequency: f0=12πLC At higher frequency: - L and C values become smaller - Smaller components → lower parasitics - Higher Q factor RC has no natural resonance — it must be forced. 3️⃣ Phase accuracy - LC tank gives: - Sharp 0° phase at resonance - RC network: - Phase depends on exact R, C - Small errors → phase mismatch At MHz: - Even 5° phase error kills RC oscillation - LC still satisfies Barkhausen easily 4️⃣ Parasitics help LC, hurt RC At high frequency: - Stray capacitance + inductance: - Form part of the LC tank - In RC: - Parasitics destroy phase response 5️⃣ Amplifier requirement - RC oscillators: - Need high gain - High gain at high freq = impossible - LC oscillators: - Need modest gain - Easy to satisfy at RF 🧠 One-line interview-ready answer (MEMORIZE) RC oscillators fail at high frequencies because resistive losses and phase errors dominate, while LC oscillators work well because energy is exchanged between L and C with low loss and accurate resonance at high frequency.. |
| Why does Hartley use inductive division and Colpitts use capacitive division instead of resistors? | Core reason (this is BIG): Resistors dissipate energy. Dividers in LC oscillators must NOT. Why this matters: - Oscillator needs energy recycling - Resistors convert signal energy → heat - That kills Q and stops oscillation Therefore: - Hartley uses inductors → energy stored magnetically - Colpitts uses capacitors → energy stored electrically Both: - Provide feedback - Do NOT dissipate energy - Preserve tank Q 🔑 One-liner: Feedback network in LC oscillators must store energy, not waste it. |
| What happens if the amplifier loads the LC tank too heavily? | First: what “loading” REALLY means (general case) You said: output shall be low impedance and next stage shall be high impedance so it acts as open circuit ✔️ Correct. That is the core rule. Loading = energy being pulled out of the previous stage When the next stage: - has low input impedance - or draws significant current …it steals energy from the previous stage instead of just “observing” the signal. Now apply this to an LC tank Think of an LC tank as: - a lossless energy storage system (ideally) - energy keeps sloshing between L ↔ C What keeps oscillation alive? - Energy lost per cycle must be very small - Amplifier should add just enough energy to compensate losses What does “amplifier loads the LC tank too heavily” mean? It means: - Amplifier input impedance is not high enough - Tank has to supply current into amplifier - Energy is drained from the tank every cycle In circuit terms: Amplifier input appears as a resistor in parallel with the LC tank. What does that cause physically? 1️⃣ Q factor drops - Tank loses energy faster - Oscillation becomes “damped” 2️⃣ Frequency becomes unstable - Tank resonance is disturbed - Frequency shifts with loading 3️⃣ Amplitude reduces - More energy is pulled out than fed back 4️⃣ Worst case: oscillation stops - Loop gain < 1 - Barkhausen fails Ultra-clear analogy (remember this) LC tank = swing Amplifier = child pushing the swing If amplifier: - pushes gently → oscillation continues - grabs the swing tightly → motion dies Heavy loading = grabbing the swing. Why buffer stages are often used This is why in real oscillators: - Emitter follower / source follower - Buffer amplifiers are placed between: - LC tank → main amplifier / output stage They provide: - High input impedance (don’t load tank) - Low output impedance (can drive next stage) One-line engineer takeaway (VERY IMPORTANT) In LC oscillators, the tank must see the amplifier as an almost open circuit, otherwise Q collapses. ✅ Can we use an op-amp buffer instead of CC / emitter follower? ✔️ YES, if the op-amp meets certain conditions ❌ NO, if it doesn’t Why an op-amp buffer can work An op-amp used as a voltage follower (unity gain) ideally has: - Very high input impedance → doesn’t load the LC tank - Very low output impedance → can drive next stage - Gain ≈ 1 → preserves waveform So conceptually, it behaves like: - Emitter follower (BJT) - Source follower (MOSFET) ➡️ Functionally correct substitute ⚠️ BUT… here’s the critical catch (this is important) 1️⃣ Bandwidth / GBW limitation LC oscillators often operate at: - Hundreds of kHz - MHz range Most general-purpose op-amps: - Have limited gain–bandwidth product - Cannot maintain unity gain at high frequency 📌 Result: - Phase shift introduced - Barkhausen phase condition violated - Oscillator becomes unstable or stops 2️⃣ Phase shift introduced by op-amp At higher frequencies: - Op-amp is not ideal - Internal poles introduce: - Phase lag - Group delay Even 10–20° phase error can kill oscillation. 3️⃣ Slew rate limitation For a sine wave: Required SR=2πfVpeak If op-amp SR is too low: - Output distorts - Effective feedback changes - Oscillation quality degrades 4️⃣ Noise + bias current interaction Op-amp input bias current + tank impedance: - Can disturb operating point - Shift frequency slightly ✅ When op-amp buffer is a GOOD choice Situation; Verdict. RC oscillators (Hz–kHz); ✅ Excellent. Wien bridge; ✅ Ideal. Audio oscillators; ✅ Perfect. LC oscillators < ~100 kHz; ✅ Often OK. LC oscillators in MHz; ⚠️ Risky. ❌ When CC / source follower is BETTER - RF oscillators - Crystal oscillators - MHz–GHz designs - Low phase noise systems Because: - Discrete followers have no GBW limit - Minimal phase shift - Much higher Q preservation 🔑 Golden design rule (write this down) Use op-amp buffers for low-frequency oscillators. Use transistor followers for high-frequency oscillators. Final mental model Buffer type; Best use. Op-amp follower; RC, Wien, audio. Emitter follower; LC, RF. Source follower; RF, crystal.. |
| Why does higher Q improve frequency stability but make oscillation startup slower? | High Q means: - Very low energy loss per cycle - Very sharp frequency selectivity Startup problem: - Noise energy is tiny - High-Q tank accepts only one exact frequency - Energy builds slowly Low Q: - Accepts more frequencies - Builds amplitude quickly - But unstable frequency 🔑 Analogy: High-Q tank is a picky listener — slow to respond, but very precise. |
| Crystal oscillator | Link : https://youtube.com/shorts/aNm9GReoqNw?si=kUJDQ4ehFtTl7y-C https://www.youtube.com/shorts/-mvZh4bBKJI https://youtube.com/shorts/cC69lnpElXg?si=lze5Gbmv5xfOM5aj https://www.youtube.com/watch?v=wcJXA8IqYl8 Piezoelectric Effect (Mechanical → Electrical) What actually happens Certain crystals (like quartz) have a non-symmetrical atomic structure. - In normal state → charges are balanced - When you apply mechanical stress (press, stretch, bend): - Positive and negative charge centers shift - This creates an electric potential (voltage) across the crystal 👉 Mechanical energy → Electrical energy Intuition (very important) Think of the crystal as a spring made of charges. - Press it → charges separate - Separate charges → voltage appears - Release → charges return → voltage disappears Simple examples - Gas lighter “click” spark - Knock sensors - Vibration sensors - Microphones (piezo mics) One-line definition (for notes) Piezoelectric effect: Mechanical stress on certain crystals produces an electric voltage. Inverse Piezoelectric Effect (Electrical → Mechanical) What happens When you apply a voltage across a piezoelectric crystal: - Electric field pulls/pushes the charged atoms - Crystal physically deforms - expands - contracts - bends (depending on cut & orientation) - 👉 Electrical energy → Mechanical motion Intuition Electric field = force on charges Charges are part of the crystal lattice So the lattice moves Very small movement — but extremely precise. Examples - Piezo buzzers - Ultrasonic transducers - Precision actuators - Inkjet printer heads One-line definition Inverse piezoelectric effect: Applying voltage to a piezoelectric crystal causes mechanical deformation. Quartz Crystal — Behaviour Summary (Signal Applied) 1️⃣ When a signal is applied at a non-resonant frequency - Applied AC voltage → inverse piezoelectric effect - Crystal does vibrate, at the same frequency as the input - But: - Vibration amplitude is extremely small - Energy does not accumulate - Mechanical inertia + stiffness oppose motion - Losses dominate immediately Result: - Only forced vibration - No ringing - No sustained motion - Electrically, crystal behaves mostly like a capacitor 👉 Signal response dies quickly 2️⃣ When a signal is applied at the mechanical resonant frequency - Applied AC voltage → inverse piezoelectric effect - Electrical forcing is in phase with natural mechanical motion - Each cycle adds energy instead of cancelling it - Mechanical vibration builds up strongly - Strong vibration → strong piezoelectric effect → electrical signal fed back Result: - Efficient electrical ↔ mechanical ↔ electrical loop (electrical signal causes mechanical vibration and mechanic vibration causes electrical charge – this goes on unless we provide voltages as there exist losses such as inertia or crystal , friction , heat losses which makes it eventually die but after a very long time as Q is very good of quartz crystals ) - Very large response for even tiny input - Crystal impedance changes sharply - Energy is stored, not dissipated immediately 👉 This is resonance 3️⃣ Why oscillation still needs external energy (losses) Even at resonance, crystal is not lossless. Main losses: - Mechanical: - Internal lattice friction - Mounting losses - Electrical: - Electrode resistance - Dielectric losses Because of these: - Stored energy slowly leaks away - If excitation is removed → vibration decays 👉 Continuous external energy is required to sustain oscillation (this is supplied by the amplifier in a crystal oscillator) 4️⃣ What “High Q” really means for a crystal - Q = (energy stored) / (energy lost per cycle) - Quartz has extremely low loss - So: - Energy decays very slowly - Resonance peak is very sharp - Only a very narrow frequency band is supported Implications of high Q: - Excellent frequency stability - Strong rejection of other frequencies - Slow startup (energy builds gradually) - Very small energy needed per cycle to sustain oscillation 5️⃣ One-line mental model (lock this in) Off resonance: crystal is forced → weak vibration → energy dies At resonance: crystal rings → energy accumulates → needs only loss compensation How a Crystal Oscillator Works (Simple & Correct) Core idea in one line A crystal oscillator uses an amplifier to continuously supply energy, and a quartz crystal to allow oscillation only at one precise frequency. Step-by-step (very simple) 1️⃣ Power is applied - Amplifier is biased and ready - Small electrical noise exists (always) 2️⃣ Noise excites the crystal - Noise contains many frequencies - Crystal responds to all → but only weakly 3️⃣ At crystal’s resonant frequency ⭐ - Inverse piezoelectric effect causes strong mechanical vibration - Mechanical vibration → piezoelectric effect → strong electrical signal - Crystal now passes feedback efficiently only at this frequency 4️⃣ Feedback loop is formed - Output of amplifier → crystal → fed back to input - At resonance: - Correct phase (0° / 360°) - Loop gain ≥ 1 👉 Barkhausen is satisfied ONLY at crystal frequency 5️⃣ Energy is replenished every cycle - Crystal has small losses (friction, resistance) - Amplifier supplies just enough energy to replace losses - Oscillation grows, then stabilizes 6️⃣ Result - Clean sine (or square) wave - Frequency = crystal’s resonant frequency - Very stable, very accurate What crystal does vs amplifier does Block; Role. Crystal; Selects ONE frequency (ultra-high Q). Amplifier; Supplies energy + maintains loop. Feedback path; Ensures correct phase. Why other frequencies die - Off-resonance: - Crystal response is weak - Loop gain < 1 - Oscillation decays Only one frequency survives. One analogy (locks it forever) Crystal = tuning fork Amplifier = hand giving tiny pushes Correct rhythm → vibration grows Wrong rhythm → nothing happens One-sentence summary (best possible) A crystal oscillator works by using an amplifier to compensate crystal losses, while the crystal’s very high-Q mechanical resonance enforces oscillation at only one precise frequency. Each quartz crystal is manufactured to have a specific resonant frequency. Different crystals → different resonant frequencies. Why different crystals have different frequencies A crystal’s resonant frequency depends on its mechanical properties, mainly: 1️⃣ Physical dimensions - Thickness - Length / width (for certain vibration modes) 👉 Thickness is the most important factor. Thinner crystal → higher frequency Thicker crystal → lower frequency 2️⃣ Cut of the crystal Quartz is cut at specific angles: - AT-cut - BT-cut - etc. The cut determines: - Vibration mode - Temperature stability - Frequency vs temperature behavior 3️⃣ Mode of vibration - Fundamental mode - Overtone modes Same crystal can support: - 1× (fundamental) - 3×, 5× (overtones) Simple intuition (best way to remember) A crystal is like a tiny tuning fork. Change its size → change its pitch. So: - 32.768 kHz crystal → thick, slow vibration - 16 MHz crystal → thinner, faster vibration Typical crystal frequencies you’ll see Application; Common frequency. Real-time clock; 32.768 kHz. Microcontrollers; 4 MHz, 8 MHz, 16 MHz. CPUs / SoCs; 25 MHz, 40 MHz. RF reference; 10 MHz. Each of these uses a different crystal. Important clarification A crystal does NOT adapt its frequency to the circuit. Instead: The circuit adapts to the crystal. If you put a 16 MHz crystal: - You get 16 MHz - Not 15.8 MHz, not 17 MHz That’s the whole power of crystals.. |
| Crystal Oscillator | Link : https://www.youtube.com/watch?v=YzcKQWwkzWs&list=PLwjK_iyK4LLCVdgBR30pSFVj-17TI_8ou&index=5 Drawbacks of RC and LC oscillators - Change in temperature - Change in power supply voltage - Change in component values When u require high stability, you go for crystal oscillator (because crystal has very high Q (around 10000 to 20000) Application of crystal oscillator Crystal oscillators are used wherever an accurate, stable, repeatable frequency is required. - Microcontrollers & Microprocessors (MOST COMMON) - What they do - Provide the system clock - Why crystal is used - CPU timing depends on clock accuracy - Instruction execution, timers, UART, SPI, I²C all derive from clock - RC / LC drift would break communication timing - Examples - Arduino (16 MHz crystal) - STM32, AVR, PIC - CPUs use crystal + PLL - 👉 Without crystal → system becomes unreliable - Real-Time Clocks (RTC) - What they do - Keep accurate time (seconds, minutes, date) - Why crystal - Uses 32.768 kHz crystal - Extremely stable - Low power - Exactly 215→ easy binary division - Examples - Watches - Wall clocks - RTC chips (DS3231, etc.) - Communication systems - What they do - Set carrier frequencies - Set baud rates - Synchronize transmitter & receiver - Why crystal - Even tiny frequency error causes: - data corruption - loss of lock - interference - Examples - UART baud rate generation - Wi-Fi, Bluetooth reference clocks - RF transceivers 👉 Crystal ensures both sides “agree on time” - Frequency Reference & Measurement - What they do - Act as a reference standard - Why crystal - High Q → precise frequency - Repeatable across systems - Examples - Frequency counters - Signal generators - Calibration equipment - 10 MHz reference oscillators - Digital Logic & Timing Circuits - What they do - Clock flip-flops - Drive state machines - Synchronize digital systems - Why crystal - Clean, jitter-free clock - Stable over temperature and voltage - Examples - FPGA clock inputs - Digital controllers - Consumer Electronics - What they do - Timing, synchronization, communication - Examples - Mobile phones - TVs - Routers - Game consoles - Set-top boxes - Almost every digital consumer product has at least one crystal. - Automotive & Industrial Systems - Why crystal - Harsh temperature environments - Long-term stability required - Safety-critical timing - Examples - Engine control units (ECU) - Sensors & controllers - Industrial automation One powerful summary (write this in your notes) Crystal oscillators are used as timing and frequency references in systems where accuracy, stability, and repeatability are critical. Here’s a clean, practical answer — no fluff, just what engineers actually use. Frequency Range of Crystal Oscillators 🔹 Overall usable range ~1 kHz to ~200 MHz (depending on crystal type and mode) But this range has clear sub-ranges with different use cases. 1️⃣ Low-frequency crystals (kHz range) Typical range - 1 kHz – 100 kHz Most common - 32.768 kHz ⭐ (watch crystal) Why used - Extremely low power - High long-term accuracy - Easy frequency division (2¹⁵) Applications - Real-time clocks (RTC) - Watches - Low-power timers 2️⃣ Medium-frequency crystals (MHz range) – MOST COMMON Typical range - 1 MHz – 30 MHz Common values - 4 MHz - 8 MHz - 16 MHz - 20 MHz - 25 MHz Why this range dominates - Fundamental mode operation - Excellent stability - Easy oscillator design Applications - Microcontrollers - Microprocessors - Digital systems - Communication clocks 3️⃣ High-frequency crystals (Overtone mode) Typical range - 30 MHz – 200 MHz ⚠️ Not fundamental mode ⚠️ Uses overtone resonance (3rd, 5th harmonic) Why overtones are needed - Very thin crystals are fragile - Fundamental resonance above ~30 MHz is impractical Applications - RF systems - High-speed clocks - Transceivers 4️⃣ Why crystals don’t usually go beyond ~200 MHz Above this: - Crystal becomes too thin - Mechanical losses increase - Manufacturing becomes difficult - Stability advantage reduces 👉 For higher frequencies: - Crystal + PLL - SAW oscillators - MEMS oscillators 5️⃣ Quick comparison table Crystal type; Frequency range; Example. Watch crystal; ~32 kHz; RTC. Fundamental crystal; 1–30 MHz; MCU clocks. Overtone crystal; 30–200 MHz; RF clocks. PLL-multiplied; Up to GHz; CPUs. One-line summary (best for notes) Crystal oscillators typically operate from a few kHz up to ~200 MHz, with most applications using 32.768 kHz or 1–30 MHz fundamental-mode crystals.. Link : https://www.youtube.com/watch?v=YzcKQWwkzWs Piezo electric effect : when external mechanical force (pressure , sound waves, etc) is provided on piezoelectric material – it generates voltage on its edges (body ends) Inverse piezo electric effect : when external volt is applied to piezo electric material, they generate mechanical force (they shrink and expand) (the mechanical stress has the same freq as the input ) Piezo electric material - Rochelle salt (max piezo electric effect but less mechanical strength – so breaks easily) - Tourmaline (least piezo electric effect but high mechanical strength) - Quartz (have piezo electric effect of Rochelle salt and mechanical strength of tourmaline) – inexpensive and easily available Electrical Equivalent Circuit (Equivalent ckt of Quartz crystal is an RLC circuit) (or Butterworth-Van Dyke (BVD) model) - Inside of the crystal (Quarts are fabricated as wafers) (Butterworth-Van Dyke (BVD) model) - Cs = Motional capacitance (depends upon Elasticity of Quarts material , Area of the plates and thickness of quarts) - Ls = Motional inductance (it shows the mechanical mass of quarts when vibrating and also depends upon the thickness of Quartz material) (value ranges from few Henries to mili Henry) - Rs = series resistance or Equivalent series resistance (it shows the real resistive loss happening in the crystal) (value varies from few ohms to 100s of kilo ohms) (it is function of crystal freq) - Cp = shunt capacitance (exist because of the electrode plates (metal contact) exist around the crystal) - Here u can see that this forms LC tank circuit hence provides freq selectivity when used in an oscillator manufacturing feedback section Motional Inductance (Lm/Ls); The effective mass (inertia) of the vibrating crystal. - Heavier vibration → harder to accelerate → like inductance - related to: - Crystal mass - Mode of vibration - Thickness (indirectly). Motional Capacitance (Cm/ Cs); The elastic compliance (springiness) of the crystal. - How easily the crystal deforms mechanically - More compliant → higher Cm - Depends on: - Elastic properties - Area - Thickness. Motional Resistance (Rm/Rs); Mechanical and electrical energy loss in the crystal. Includes: - Internal friction of lattice - Mounting losses - Acoustic losses - Electrode resistance. Parallel (Shunt) Capacitance (C₀/Cp); Capacitance between crystal electrodes and holder. - Exists even when crystal is not vibrating - Purely electrical - NOT part of mechanical motion. - The crystal has 2 resonating frequencies - Series resonant frequency ( fs ) - Parallel resonant frequency ( fp ) Understanding the graph - So, when using quartz crystal for any application one must decide which resonant frequency is to be used - Here you can see the impedance offered at series resonant freq is the minimum and at parallel resonant freq it is maximum (Same as series RLC – low impedance and parallel RLC – high impedance) - And between series and parallel resonant freq – impedance will be inductive – while you go above parallel resonant freq or below series resonant freq – impedance will be capacitive - Parallel resonant freq > series resonant freq - The resonant freq is determined by how crystal is cut during the fabrication and also depends upon the thickness of the crystal - f ∝ 1Thickness [ lesser the thickness of the crystal , larger the freq ] - But when you go above certain freq – there is no possibility to reduce the thickness of the crystal (hence, crystal is operated at overtone freq instead of fundamental frequencies) [in simple terms – overtone freq = odd harmonics of the fundamental freq] – so, operating crystals at overtone frequencies all to achieve 100s of MHz freq in oscillator - Series resonant freq fs= 12π LsCs - Parallel resonant freq fp= 12π LsCeq Ceq= CpCsCp+ Cs Crystal oscillator structure - Crystal provides 180-degree phase shift and amplifier provides 180-degree phase shift - Crystal can be operated at series or parallel resonant frequencies - Crystal operated at series resonant frequency - Transistor is used as amplifier (in CE configuration) - Crystal is connected as feedback structure from collector to base - At series resonance – impedance offered by crystal will be minimum (as shown in prev graph of impedance vs freq) (feedback provided from output to input side will be max – as output impedance of crystal is minimum and input impedance of amplifier is high – so max signal transfer takes place and no loading effect) - By adjusting the gain, we can have sustained oscillation - Crystal operated at parallel resonant frequency - There is two config where crystal is used at parallel resonant freq - Colpitts oscillator - Pierce oscillator Colpitts oscillator Note : this is operated at a freq between series resonant freq and parallel resonant freq (thus crystal will act as inductive reactance) [ hence can be perfectly fitted in Colpitts oscillator as inductor] - The crystal (instead of inductor) and combination of c1 & c2 will act as a tank circuit - Also provides freq selectivity Pierce oscillator - Here , CMOS inverter is used as an amplifier - It works in the linear region (as shown in the image) - Here the crystal as inductor and C1 and C2 together forms LC tank - Also, crystal having high Q provide precise freq selectivity - Rf is the feedback resistor (for CMOS as amplifier)(value of Rf depends upon the operating freq) (usually more than 1 Mega ohm) - The series resistance Rs reduces the overtone oscillation (period odd harmonics) and also improves the startup response of the oscillator - Output of this ckt is “sine wave” – then followed by some ckt to convert it to square wave(thus most often used in digital ckt) - This same ckt is used into microcontroller and microprocessors (but Here Rs and Rf are internal to microcontroller – so by just connecting crystal and capacitor – one can generate stable clock of desired freq) Note : - whenever the crystal is used in parallel resonant mode(talking about pierce oscillator) – manufacturer provide the load capacitance for the specific freq) - so, while designing crystal oscillator for any desired freq – this detail is quite crucial (also note – these are the oscillators which uses parallel resonant freq mode) - so , equivalent capacitance (seen by crystal) should always match the load capacitance provided in datasheets, so crystal can operate at specific freq (if Ceq is over or lesser than the load capacitance mentioned – oscillator freq changes) - Ceq calculation (seen by the crystal) - Ceq when input and output capacitance of CMOS inverter is not considered Ceq= C1* C2C1+ C2 - Ceq when input and output capacitance of CMOS inverter is considered Ceq= ( C1+ Cin)* ( C2+ Cout)C1+ C2+ Cin+ Cout - Ceq when input and output capacitance of CMOS inverter is considered and also the stray capacitance Ceq= ( C1+ Cin)* ( C2+ Cout)C1+ C2+ Cin+ Cout + Cs - So, to operate the crystal at desired freq ( CLoad= Ceq ) Parameters that are needed to be considered while selecting crystal for a specific oscillator - Crystal frequency (freq at which crystal is going to get operated) - Drift in the crystal freq over the time (in PPM) - Mode of operation (fundamental or overtone) - Temperature stability - Drive level (power dissipation) across the crystal (select crystal such that drive level is less than that of rated levels) Pierce Oscillator (with CMOS Inverter Explained) 1️⃣ What is a Pierce oscillator (one-line) A Pierce oscillator is a crystal oscillator that uses a CMOS inverter as an amplifier and a quartz crystal as a highly selective feedback element to generate a stable clock frequency. This is the most common oscillator used in microcontrollers. 2️⃣ The core blocks in a Pierce oscillator A Pierce oscillator has four essential parts: - CMOS inverter → amplifier + phase shift - Quartz crystal → frequency selection (high Q) - Two capacitors (C1, C2) → phase + load capacitance - Bias resistor (Rf) → sets inverter in linear region Let’s understand each properly. 3️⃣ How a CMOS inverter works as an amplifier (VERY IMPORTANT) Normal digital inverter Normally, a CMOS inverter is used like this: - Input LOW → Output HIGH - Input HIGH → Output LOW That’s switching mode, not amplification. In oscillator: inverter is used in linear region The trick is this resistor Rf (very important). - Rf connects output → input - It biases the inverter input to around VDD/2 - At VDD/2: - NMOS is partially ON - PMOS is partially ON 📌 This is the high-gain region of the inverter. What happens now? Small AC changes at the input: - Cause large AC changes at the output So the inverter behaves like: A high-gain inverting amplifier Key point (lock this 🔒) A CMOS inverter is an excellent small-signal amplifier when biased at its switching threshold. That’s why it’s used. 4️⃣ Phase condition (Barkhausen link) For oscillation, we need: - Total loop phase = 0° (or 360°) - Loop gain ≥ 1 The inverter provides: - 180° phase shift So the feedback network (crystal + capacitors) must provide: - another 180° 5️⃣ Role of the quartz crystal (the heart ❤️) The quartz crystal: - Is a mechanical resonator - Has extremely high Q - Acts like a very sharp band-pass filter Electrically, near resonance: - It behaves like a frequency-selective impedance - Only one frequency passes easily 📌 The crystal decides the frequency, not the inverter. 6️⃣ What do the two capacitors (C1, C2) do? These capacitors are connected: - One from crystal to ground (C1) - One from inverter input/output node to ground (C2) They do three things: 1️⃣ Provide required phase shift (basically both caps and crystal (acting as inductor due to freq in the Fs < f < Fp range- forms a LC tank and (2 caps are used to provide phase inversion)) Together with the crystal, they complete the feedback phase requirement. 2️⃣ Set crystal load capacitance The crystal sees an effective capacitance: CL≈C1C2C1+C2+Cstray This matters because: - Crystal frequency is specified for a given CL - Wrong CL → frequency error 3️⃣ Stabilize oscillation They limit loop gain and prevent overdrive of the crystal. 7️⃣ How oscillation actually starts (step-by-step) This is where everything comes together. Step 1: Noise kick - Thermal noise exists everywhere - Tiny voltage fluctuation appears at inverter input Step 2: Amplification - Inverter (biased in linear region) amplifies this noise Step 3: Frequency selection - Crystal allows only its resonant frequency - All other frequencies are heavily attenuated Step 4: Positive feedback - Selected frequency is fed back - Phase condition satisfied - Signal grows each cycle Step 5: Amplitude stabilization - Nonlinearity of inverter limits amplitude - Oscillation settles at steady value 📌 Result: - Stable, precise oscillation 8️⃣ What waveform is present where? Important practical detail 👇 Location; Waveform. Across crystal; Nearly sine wave. Inverter output; Distorted / square-like. MCU clock input; Clean digital square wave. Why? - Crystal filters harmonics - Inverter re-squares the signal for logic 9️⃣ Why Pierce oscillator is so popular Advantages - Very few components - Extremely stable frequency - Low power - Uses standard CMOS inverter - Easy to integrate on chip That’s why: - Almost all MCUs use it - CPUs, SoCs use variants of it 🔟 One clean note-ready explanation ✍️ You can paste this directly: In a Pierce oscillator, a CMOS inverter is biased in its linear region using a feedback resistor, allowing it to function as a high-gain inverting amplifier. A quartz crystal placed in the feedback path provides extremely selective frequency control due to its high Q factor, while two capacitors set the required phase shift and effective load capacitance. Noise initiates oscillation, and only the crystal’s resonant frequency is reinforced through positive feedback, resulting in a stable and precise clock signal. 🧠 Final mental picture (lock this) - Inverter → provides gain + inversion - Crystal → decides frequency - Capacitors → tune & stabilize - Resistor → biases inverter correctly 👉 The inverter wants to oscillate at many frequencies. 👉 The crystal allows only one. That’s the Pierce oscillator — simple, elegant, and powerful.. |
| Ring Oscillator (RO) | Link : https://www.youtube.com/watch?v=59MzBHzdbVs&t=1s Link : (IMP for chip designing insights) : https://www.youtube.com/watch?v=V7Tw61cIPPo&t=10s Ring Oscillator = RO - Ring oscillator generates square wave of specific freq - Odd number of inverters are used to form this (because if we connect odd numbers, we get output which is diff from input - Example - If we use even number of inverters - For 2 inverter stages : If input is ‘0’ -> op after 1st inverter = ‘1’ and op after 2nd inverter = ‘0’ - So that is why we don’t use even number of inverters - If we use odd number of inverters - For 3 inverter stages : if input is ‘0’ op after 1st stage = ‘1’ and op after 2nd stage = ‘0’ and op after 3rd stage = ‘1’ - Thus, we get different output from the input – this is what we need (hence we use odd number of inverter stages) - Now the main concern is that (every inverter has some propagation delay tp ) – time taken by output to appear for relative instance of input - We need ring oscillator to satisfy two conditions - Output is diff logic from input - Propagation delay occurrence - Now how this ring oscillator generates SQUARE WAVE - Let suppose input is logic ‘1’ - Here output will change from x logic level to y logic level after 3tp - And change back again from Y to X after 3tp - This is how we get square wave - We fed logic ‘1’ to a close box – it gives back another logic state ’0’ and take 3tp time to transition - And same happens for logic state ‘0’ to ‘1’ - This is how in 6tp cycle a square wave is generated (time period of square wave = 6tp ) - Frequency = f = 16tp - if we consider N odd number of inverter stages - freq=f= 12N tp - so, by changing the number of inverter stages one can change the oscillator freq - example: - propagation delay of inverter = tp = 1ns - required oscillation freq = 100MHz - then from above mentioned formula – number of stages = 5 Applications of Ring oscillators - (Voltage controlled Oscillator) : Vdd affects the freq of oscillations (IMP) - Propagation delay is function of Vdd (for more details – read propagation delay sections of MOSFET) - 1️⃣Full Voltage Swing (Vdd and 0) & Its Effect - Full swing input = logic “0” = 0 V (GND), logic “1” = Vdd. - Why it matters: The MOSFET turns on more strongly when the gate–source voltage (Vgs) is fully high (for NMOS) or fully low (for PMOS). - Effect on propagation delay: - If input doesn’t reach full Vdd → MOSFET doesn’t fully turn on → higher R_on → slower charging/discharging of the load capacitor (CL). - Result: tₚ increases. - 2️⃣ Example - Suppose Vdd = 5 V. - Ideal input swings from 0 → 5 V → NMOS fully enhanced when Vin = 5 V. - If input only swings 0 → 3 V → NMOS partially enhanced → less current → slower output transition. - Practical Rule - To minimize propagation delay: drive CMOS inputs with full rail-to-rail voltage (0 → Vdd). - This is why CMOS outputs are usually directly connected to CMOS inputs (they are rail-to-rail). - So , as the Vdd ↑ tp ↓ frequency of oscillator increases - So , as the Vdd ↓ tp ↑ frequency of oscillator decreases - This is why RING oscillators are also referred as Voltage controlled oscillators - RANDOM number Generator - Propagation delay is function of Vdd(or say biasing voltage) and temperature as well - When these parameter changes– and thus the freq of oscillator will change -– we observe jitter(Jitter is short-term variation in the timing of signal edges.) in the waveform - Jitter is the fluctuations in the exact timings of the transition in ring oscillator output- we can use this jitter to generate random numbers (so for that we can sample the op of oscillator at specific time – which will generate random bits of 1 and 0s – this way it is possible to generate random number) - But jitter due to parameter variation can be same if sampled periodically - Hence more than 1 ring oscillators are used (each having different specs – , different number of stages , width of gate , etc.) – so provide different jitter for temp and voltage variations – and we XOR them - Temperature and Vdd(biasing voltage) measurement in ICs - We know that propagation delay increases when temp of Vdd changes - When chip is manufactured (at const temp) – freq is measured and used as a reference/calibrated freq) - So, when Temp ↑ freq of ring oscillator ↓ (considering no changes in Vdd) [we measure freq and identify temp – keeping other parameters as const ] - Same way , if temp is kept constant and freq changes – the change in the freq is used to identify Vdd/biasing volt changes - If both Vdd and temp is changing same time it is not possible to measure any of them - It is very low-cost solution - Benchmarking the new Hardware technology - Using new hardware – a ring oscillator of specific stages are designed - Oscillation freq of the ring oscillator is measured - f= 12N tp - If freq and number of stages are known – propagation delay of inverter can be estimated - From the propagation delay of inverter, we can get insights of speed and performance of the new hardware technology - So , this is how ring oscillators are used to benchmark new hardware technologies - Wafer Testing - During the wafer testing , ring oscillators are embedded in wafer as a test structure - By doing so – it is possible to measure the various aspects of the manufacturing process - So if there is any deviation in manufacturing process – it can affect the transistor behaviour/mosfet and in turn affecting the propagation delay (so, basically by measuring the freq of oscillation, it is ensured that the process is followed properly and has no errors) - Aging monitors - Transistors age (BTI, HCI) - Delay increases over time - RO frequency slowly drops - 👉 Track frequency drift → detect aging PVT = Process + Voltage + Temperature Effect Process (P) - Fast silicon → less delay → higher RO(ring oscillator) freq - Slow silicon → more delay → lower RO(ring oscillator) freq Voltage (V) - Higher VDD → faster ≅higher freq - Lower VDD → slower≅low freq Temperature (T) - Higher temp → mobility ↓ → slower ≅low freq - Lower temp → faster ≅higher freq Typical frequency ranges (by context) 🔹 Older / long-channel CMOS (≥180 nm) - ~10 kHz – 100 MHz - Large inverter delay - Used mainly for teaching, characterization 🔹 Mainstream CMOS (90 nm – 28 nm) - ~10 MHz – 1 GHz - Common for: - PVT monitors - On-chip sensors - Adaptive voltage scaling 🔹 Advanced nodes (16 nm – 5 nm) - ~500 MHz – 5+ GHz - Very small gate delay - Used in: - High-speed silicon characterization - Aging and variability studies What sets the frequency? (intuition) f≈12⋅N⋅tpd Where: - N= number of inverters (odd) - tpd= inverter delay Key levers: - ↓ inverter delay → ↑ frequency - ↓ number of stages → ↑ frequency - ↑ VDD → ↓ delay → ↑ frequency |
| WHY ring oscillators are unstable | Ring oscillators are unstable because: 1️⃣ No frequency-selective element - No LC tank - No crystal - Frequency is set only by gate delays 2️⃣ Propagation delay is highly sensitive Delay depends on: - Supply voltage (Vdd) - Temperature - Process corner (FF / TT / SS) - Load capacitance - Noise Small change → large frequency shift 3️⃣ Low Q (almost zero Q) - No energy storage - No resonance - No frequency “locking” 4️⃣ High phase noise / jitter - Every inverter adds timing uncertainty - Noise accumulates stage by stage 📌 So: Ring oscillators are delay-based, not resonance-based. That’s why they drift. |

| CKT designing in LTspice |  |
| --- | --- |
| RC Phase shift oscillator | Circuit : - As per RC phase shift oscillator min theoretical gain of opAmp shall be 29 (but practically >30-32) - Gain here = - RfR1 = -35 - Freq of oscillation = f= 12πRC2N = 12πRC6=650Hz - Period of oscillation signal = 1/650 = 1.53ms Achieved Result - Achieved freq is 648 Hz - Range of amplitude is 11 to -11v (which cannot be calculated in case of RC – that depends on the noise getting amplified which is uncertain) – so we don’t care about amplitude [ this is sine waveform] “Skip initial operating point solution” — what it really means 🔹 Normally (option unchecked) LTspice does this before starting transient simulation: - Computes DC operating point - Capacitors → treated as open circuits - Inductors → treated as short circuits - Finds a perfectly stable DC solution - Uses that as the starting condition for .tran 👉 This is great for amplifiers, regulators, power supplies. 🔹 When you check “Skip initial operating point solution” LTspice: - Does NOT solve the DC operating point - Starts the transient simulation directly - Uses: - Capacitor initial voltages = 0 V - Inductor currents = 0 A - Small numerical noise 👉 This tiny “disturbance” is what kicks oscillators into life.. |
| Wein bridge oscillator | Consider R2 = R3 = 160 ohm and C1 = C2 = 0.1uf Frequency = 10KHz |

| Heartley Oscillator | Link : https://youtu.be/QkI5vJg3MFI?si=f1nnQofF__k5v4Bn Circuit Configuration Result - Here BJT is in linear region (active region ) is used as amplifier - For active region (4 condition are to be met) - NPN transistor - Base–Emitter junction → forward biased & Base–Collector junction → reverse biased - VBE ≈0.6-0.7v - VC> VB> VE - VCE> VCEsat (typically>0.2v) - If this is true – transistor is in active region - Q point (Quiescent point – where proper amplification takes place) - For CE config ( Ic , VCE ) - VB= R2R1+ R2 Vcc 0.9523 - VE= VB- VBE 0.2523 - IE= VERE ≈ IC - VC= Vcc- ICRC (Here instead of a resistor an inductor is used thus instead of RC ,ZL will be used→ ZL=2πfL - VCE = VC- VE Why an Inductor is Used Instead of RCin Oscillators (CE Stage) ? Core Reason (one line) An inductor is used as the collector load instead of a resistor because it provides very high AC impedance at the oscillation frequency (high gain) while causing negligible DC power loss, which is essential for efficient and stable oscillation. - The amplifier (BJT in CE config) is used which provides 180-degree phase shift - So, to achieve another 180-degree phase shift – in feedback LC tank ckt – centre tapped or 2 separate inductors are used (basically feedback is taken from -ve terminal (or AC ground or centre tapped) and another terminal is lower point of the tank (making signal appear reverse polarity ) (which is 180 degree out of phase) – so 180 + 180 = 360 (Barkhausen criteria satisfied) - Here - β= L1L2= 2.3u0.23=0.1= 110 - So, A must be 10 in order to achieve loop gain = Aβ=1 - Actual gain of amplifier |
| --- | --- |
| Colpitts oscillator | Note : Check how to pick L & C for oscillator (refer LC oscillator section notes of this word file) LEARN HEARTLY Oscillator first if u r visiting this - Assume required freq is 1 MHz - Choosing good L & C (larger C and smaller L) Circuit |
| Crystal oscillator in general | Steps to generate Crystal symbol - Open schematic - Form below shown schematic (save it as ‘XTAL_10MHz_BVD’) Here for 10 MHz Xc and Xl is almost same (so this is resonance freq) (Xc=796178 and Xl = 795048) - Here (XTAL1 and XTAL2 is netlist – defined as input and output respectively) - Then go to Hierarchy open or generate this sheet’s symbol click ok ok - You will see below shown image - Save it (it will get saved as ‘XTAL_10MHz_BVD.asy’) - Close it Using it - Open new schematic - Then go to Edit component u will see the name of the symbol as shown below - You will see symbol on screen as shown below - Now lets check that weather it resonates at 10MHz or not (by building a test ckt and testing it) - Create ckt like this - Right click on voltage source V1 and add details as shown (AC amplitude = 1 , DC Amplitude = 0 ) - Add spice directive “.ac dec 500 1Meg 50Meg” - Click RUN - Click on the point between Resistor R1 and crystal X1 (u will see something like this) - Fp = 10.046 MHz - Fs = 10.000 MHz Zoomed in ✅ What plot proves 1️⃣ Deep sharp dip at ~10 MHz - That narrow deep notch is series resonance (fs) - At this point: - Ls and Cs cancel - Crystal impedance ≈ Rs only - Maximum current flows ✔ This confirms your Lm–Cm calculation is correct 2️⃣ Slight recovery just above fs - That small rise after the dip corresponds to parallel resonance (fp) - Caused by interaction of motional branch + Cp - fp > fs (always true for real crystals) 3️⃣ Shape tells you the Q is high - Very narrow dip = high Q - That means: - Rs is reasonable - Values are physically realistic - Model is stable for oscillators This is exactly how real quartz behaves. |
| Pierce Oscillator | - Develop below shown ckt in LTspice - Add - Then click on RUN. You will see below shown graph |